Espressif Systems IOT Team

Copyright (c) 2015

# ESP8266 SDK API Guide

Version 1.0.1

Disclaimer and Copyright Notice

Information in this document, including URL references, is subject to change without notice.

THIS DOCUMENT IS PROVIDED AS IS WITH NO WARRANTIES WHATSOEVER, INCLUDING ANY WARRANTY OF MERCHANTABILITY, NONINFRINGEMENT, FITNESS FOR ANY PARTICULAR PURPOSE, OR ANY WARRANTY OTHERWISE ARISING OUT OF ANY PROPOSAL, SPECIFICATION OR

SAMPLE. All liability, including liability for infringement of any proprietary rights, relating to use of information in this document is disclaimed. No licenses express or implied, by estoppel or otherwise, to any intellectual property rights are granted herein.

The Wi-Fi Alliance Member Logo is a trademark of the Wi-Fi Alliance.

All trade names, trademarks and registered trademarks mentioned in this document are property of their respective owners, and are hereby acknowledged.

Copyright © 2015 Espressif Systems Inc. All rights reserved.

Table of Content

- Preambles	11

- Overview	11

- Application Programming Interface (APIs)	13

  - Timer	13

    - os_timer_arm	13

    - os_timer_disarm	13

    - os_timer_setfn	14

  - System APIs	14

    - system_restore	14

    - system_restart	15

    - system_timer_reinit	15

    - system_init_done_cb	15

    - system_get_chip_id	16

    - system_deep_sleep	16

    - system_deep_sleep_set_option	17

    - system_set_os_print	17

    - system_print_meminfo	18

    - system_get_free_heap_size	18

    - system_os_task	18

    - system_os_post	19

    - system_get_time	20

    - system_get_rtc_time	20

    - system_rtc_clock_cali_proc	20

    - system_rtc_mem_write	21

    - system_rtc_mem_read	22

    - system_uart_swap	22

    - system_get_boot_version	23

    - system_get_userbin_addr	23

    - system_get_boot_mode	23

    - system_restart_enhance	24

    - system_update_cpu_freq	24

    - system_get_cpu_freq	24

  - SPI Flash Related APIs	25

    - spi_flash_get_id	25

    - spi_flash_erase_sector	25

    - spi_flash_write	26

    - spi_flash_read	26

  - WIFI Related APIs	28

    - wifi_get_opmode	28

    - wifi_get_opmode_default	28

    - wifi_set_opmode	28

    - wifi_set_opmode_current	29

    - wifi_station_get_config	29

    - wifi_station_get_config_default	30

    - wifi_station_set_config	30

    - wifi_station_set_config_current	30

    - wifi_station_connect	31

    - wifi_station_disconnect	31

    - wifi_station_get_connect_status	32

    - wifi_station_scan	32

    - scan_done_cb_t	33

    - wifi_station_ap_number_set	33

    - wifi_station_get_ap_info	34

    - wifi_station_ap_change	34

    - wifi_station_get_current_ap_id	34

    - wifi_station_get_auto_connect	35

    - wifi_station_set_auto_connect	35

    - wifi_station_dhcpc_start	36

    - wifi_station_dhcpc_stop	36

    - wifi_station_dhcpc_status	36

    - wifi_station_set_reconnect_policy	37

    - wifi_softap_get_config	37

    - wifi_softap_get_config_default	38

    - wifi_softap_set_config	38

    - wifi_softap_set_config_current	38

    - wifi_softap_get_station_info	39

    - wifi_softap_free_station_info	39

    - wifi_softap_dhcps_start	40

    - wifi_softap_dhcps_stop	40

    - wifi_softap_set_dhcps_lease	41

    - wifi_softap_dhcps_status	42

    - wifi_softap_dhcps_set_offer_option	42

    - wifi_set_phy_mode	43

    - wifi_get_phy_mode	44

    - wifi_get_ip_info	44

    - wifi_set_ip_info	44

    - wifi_set_macaddr	45

    - wifi_get_macaddr	46

    - wifi_set_sleep_type	46

    - wifi_get_sleep_type	47

    - wifi_status_led_install	47

    - wifi_status_led_uninstall	48

    - wifi_set_broadcast_if	48

    - wifi_get_broadcast _if	49

    - wifi_set_event_handler_cb	49

  - Upgrade (FOTA) APIs	52

    - system_upgrade_userbin_check	52

    - system_upgrade_flag_set	52

    - system_upgrade_flag_check	52

    - system_upgrade_start	53

    - system_upgrade_reboot	53

  - Sniﬀer Related APIs	54

    - wifi_promiscuous_enable	54

    - wifi_promiscuous_set_mac	54

    - wifi_set_promiscuous_rx_cb	55

    - wifi_get_channel	55

    - wifi_set_channel	55

  - smart config APIs	56

    - smartconfig_start	56

    - smartconfig_stop	57

    - get_smartconfig_status	57

  - SNTP APIs	58

    - sntp_setserver	58

    - sntp_getserver	58

    - sntp_setservername	59

    - sntp_getservername	59

    - sntp_init	59

    - sntp_stop	60

    - sntp_get_current_timestamp	60

    - sntp_get_real_time	60

    - SNTP Example	60

- TCP/UDP APIs	62

  - Generic TCP/UDP APIs	62

    - espconn_delete	62

    - espconn_gethostbyname	62

    - espconn_port	63

    - espconn_regist_sentcb	64

    - espconn_regist_recvcb	64

    - espconn_sent_callback	65

    - espconn_recv_callback	65

    - espconn_sent	65

  - TCP APIs	66

    - espconn_accept	66

    - espconn_secure_accept	66

    - espconn_regist_time	67

    - espconn_get_connection_info	68

    - espconn_connect	68

    - espconn_connect_callback	69

    - espconn_set_opt	69

    - espconn_clear_opt	70

    - espconn_set_keepalive	70

    - espconn_get_keepalive	71

    - espconn_disconnect	72

    - espconn_regist_connectcb	73

    - espconn_regist_reconcb	73

    - espconn_regist_disconcb	74

    - espconn_regist_write_finish	74

    - espconn_secure_set_size	74

    - espconn_secure_get_size	75

    - espconn_secure_connect	75

    - espconn_secure_sent	76

    - espconn_secure_disconnect	76

    - espconn_tcp_get_max_con	77

    - espconn_tcp_set_max_con	77

    - espconn_tcp_get_max_con_allow	77

    - espconn_tcp_set_max_con_allow	78

    - espconn_recv_hold	78

    - espconn_recv_unhold	79

  - UDP APIs	79

    - espconn_create	79

    - espconn_igmp_join	79

    - espconn_igmp_leave	80

  - mDNS APIs	80

    - espconn_mdns_init	80

    - espconn_mdns_close	81

    - espconn_mdns_server_register	81

    - espconn_mdns_server_unregister	82

    - espconn_mdns_get_servername	82

    - espconn_mdns_set_servername	82

    - espconn_mdns_set_hostname	82

    - espconn_mdns_get_hostname	83

    - espconn_mdns_disable	83

    - espconn_mdns_enable	83

- Application Related	85

  - AT APIs	85

    - at_response_ok	85

    - at_response_error	85

    - at_cmd_array_regist	85

    - at_get_next_int_dec	86

    - at_data_str_copy	86

    - at_init	87

    - at_port_print	87

    - at_set_custom_info	87

    - at_enter_special_state	88

    - at_leave_special_state	88

    - at_get_version	88

    - at_register_uart_rx_intr	89

  - Related JSON APIs	90

    - jsonparse_setup	90

    - jsonparse_next	90

    - jsonparse_copy_value	90

    - jsonparse_get_value_as_int	91

    - jsonparse_get_value_as_long	91

    - jsonparse_get_len	91

    - jsonparse_get_value_as_type	92

    - jsonparse_strcmp_value	92

    - jsontree_set_up	92

    - jsontree_reset	93

    - jsontree_path_name	93

    - jsontree_write_int	94

    - jsontree_write_int_array	94

    - jsontree_write_string	94

    - jsontree_print_next	95

    - jsontree_find_next	95

- Definition of Structures	96

  - Timer	96

  - WiFi Related Structures	96

    - Station Related	96

    - soft-AP related	96

    - scan related	97

    - WiFi event related structure	97

    - smart config structure	100

  - JSON Related Structure	100

    - json structure	100

    - json macro definition	101

  - espconn parameters	102

    - callback function	102

    - espconn	102

- Peripheral Related Drivers	105

  - GPIO Related APIs	105

    - PIN Related Macros	105

    - gpio_output_set	105

    - GPIO input and output macro	106

    - GPIO interrupt	106

    - gpio_pin_intr_state_set	106

    - GPIO Interrupt Handler	107

  - UART Related APIs	107

    - uart_init	107

    - uart0_tx_buffer	108

    - uart0_rx_intr_handler	108

  - I2C Master Related APIs	109

    - i2c_master_gpio_init	109

    - i2c_master_init	109

    - i2c_master_start	109

    - i2c_master_stop	109

    - i2c_master_send_ack	110

    - i2c_master_send_nack	110

    - i2c_master_checkAck	110

    - i2c_master_readByte	111

    - i2c_master_writeByte	111

  - PWM Related	111

    - pwm_init	111

    - pwm_start	112

    - pwm_set_duty	112

    - pwm_set_freq	112

    - pwm_get_duty	113

    - pwm_get_freq	113

- Appendix	114

  - ESPCONN Programming	114

    - TCP Client Mode	114

    - TCP Server Mode	114

  - RTC APIs Example	115

  - Sniﬀer Structure Introduction	116

  - ESP8266 ADC & VDD33	119

Applications Scenario 1：Measure power voltage of VDD3P3 pin 3 and 4	119

Applications Scenario 2：Input Voltage of TOUT pin 6	119

Notice	120

  - ESP8266 soft-AP and station channel configuration	120

# Preambles

ESP8266 WiFi SoC предлагает полное и автономное сетевое решение Wi-Fi; его можно использовать для размещения приложения или для разгрузки сетевых функций Wi-Fi от другого процессора приложений. Когда ESP8266 размещает приложение, оно загружается непосредственно с внешней флэш-памяти. В него встроен кэш для повышения производительности системы в таких приложениях. В качестве альтернативы, беспроводной доступ в Интернет может быть добавлен в качестве адаптера Wi-Fi к любой конструкции на базе микроконтроллера с простым подключением через интерфейс UART или мостовой интерфейс CPU AHB.

ESP8266EX является одним из самых интегрированных чипов WiFi в отрасли; он объединяет антенные переключатели, радиочастотный балун, усилитель мощности, малошумящий приемный усилитель, фильтры, модули управления питанием, требует минимальной внешней схемы, а все решение, включая входной модуль, спроектировано так, чтобы занимать минимальную площадь печатной платы.

ESP8266EX также интегрирует улучшенную версию 32-разрядного процессора Tensilica серии L106 Diamond со встроенной памятью SRAM, а также функциями Wi-Fi. ESP8266EX часто интегрируется с внешними датчиками и другими устройствами для конкретных приложений через GPIO; Коды для таких приложений приведены в примерах в пакете SDK.

Сложные функции системного уровня включают быстрое переключение контекста спящего режима/бодрствования для энергоэффективной VoIP, адаптивное смещение радиосигнала для работы с низким энергопотреблением, расширенную обработку сигналов, а также функции подавления сигналов и сосуществования радио для подавления помех обычной сотовой связи, Bluetooth, DDR, LVDS, ЖК-дисплея.

SDK на базе платформы ESP8266 IoT предлагает пользователям простой, быстрый и эффективный способ разработки IoT-устройств. В этом руководстве по программированию представлен обзор пакета SDK, а также подробные сведения об API. Он написан для разработчиков встроенного программного обеспечения, чтобы помочь им программировать на ESP8266 платформе IoT.

# Overview

SDK предоставляет набор интерфейсов для функций приема и передачи данных на уровне Wi-Fi и TCP/ IP, чтобы программисты могли сосредоточиться на разработке приложений на высоком уровне. Пользователи могут легко использовать соответствующие интерфейсы для реализации приема и передачи данных.

Все сетевые функции на IoT-платформе ESP8266 реализованы в библиотеке, и не прозрачны для пользователей. Вместо этого пользователи могут инициализировать интерфейс в user_main.c.

void user_init(void) является методом по умолчанию. Пользователи могут добавлять в интерфейс такие функции, как инициализация прошивки, настройка сетевых параметров и инициализация таймера.

Пакет SDK предоставляет API для обработки JSON, и пользователи также могут использовать самостоятельно определенные типы данных для их обработки .

Примечание:

      - При использовании однопоточного пакета SDK, не относящегося к ОС, любая задача не должна занимать процессор слишком долго;

        - Если задача занимала процессор в течение длительного времени, ESP8266 не может накормить собаку, вызовет сброс сторожевого таймера;

        - Задача не должна занимать процессор более 10 мс, в противном случае может произойти разрыв Wi-Fi соединения.

      - Мы рекомендуем использовать таймер для периодической проверки.

      - При использовании SDK, не относящегося к ОС, не вызывайте функцию, определенную с помощью обработчика прерываний ICACHE_FLASH_ATTR in.

      - Мы предлагаем использовать RTOS SDK, RTOS может планировать различные задачи.

# Application Programming Interface (APIs)

## Timer

Timer can be found: /esp_iot_sdk/include/osapi.h.

Please be noted that os_timer_arm cannot be invoked during interruption.

For the same timer, os_timer_arm cannot be invoked repeatedly. os_timer_disarm should be invoked first.

os_timer_setfn can only be invoked when the timer is not enabled, i.e., before os_timer_arm or after os_timer_disarm.

Timer callback may not be precisely executed at the right time, it depends on priority.

### os_timer_arm

Function:

Initialising the timer.

Prototype:

void os_timer_arm ( ETSTimer *ptimer, uint32_t milliseconds, bool repeat_flag

)

Parameters:

ETSTimer *ptimer : Timer structure

uint32_t milliseconds : Timing, Unit: millisecond, the maximum value is 6871947ms

bool repeat_flag : Whether the timer will be invoked repeatedly or not

Return:

null

### os_timer_disarm

Function:

Disarm timer

Prototype:

void os_timer_disarm (ETSTimer *ptimer)

Parameters:

ETSTimer *ptimer : Timer structure

Return:

null

### os_timer_setfn

Function:

Set timer callback function

Prototype:

void os_timer_setfn( ETSTimer *ptimer, ETSTimerFunc *pfunction, void *parg

)

Parameters:

ETSTimer *ptimer : Timer structure

TESTimerFunc *pfunction : timer callback function void *parg : callback function parameter

Return:

null

## System APIs

### system_restore

Function:

Reset to default settings of following APIs ：wifi_station_set_auto_connect, wifi_set_phy_mode, wifi_softap_set_config related, wifi_station_set_config related, wifi_set_opmode, and APs information recorded by #define AP_CACHE

Prototype:

void system_restore(void)

Parameters: null

Return:

null

### system_restart

Function:

Restart

Prototype:

void system_restart(void)

Parameters: null

Return:

null

### system_timer_reinit

Function:

Reinitiate the timer when you need to use microsecond timer

Notes:

- Define USE_US_TIMER;

- Put system_timer_reinit at the beginning of user_init , in the first sentence.

Prototype:

void system_timer_reinit (void)

Parameters: null

Return:

null

### system_init_done_cb

Function:

Call this API in user_init to register a system-init-done callback.

Note:

wifi_station_scan has to be called after system init done and station enable.

Prototype:

void system_init_done_cb(init_done_cb_t cb)

Parameter:

init_done_cb_t cb : system-init-done callback

Return:

null

Example:

void to_scan(void) { wifi_station_scan(null,scan_done); } void user_init(void) {

wifi_set_opmode(STATION_MODE); system_init_done_cb(to_scan);

}

### system_get_chip_id

Function:

Get chip ID

Prototype:

uint32 system_get_chip_id (void)

Parameters: null

Return:

Chip ID

### system_deep_sleep

Function:

Configures chip for deep-sleep mode. When the device is in deep-sleep, it automatically wakes up periodically; the period is configurable. Upon waking up, the device boots up from user_init.

Prototype:

void system_deep_sleep(uint32 time_in_us)

Parameters:

uint32 time_in_us : during the time (us) device is in deep-sleep

Return:

null

Note:

Hardware has to support deep-sleep wake up (XPD_DCDC connects to EXT_RSTB with 0R).

system_deep_sleep(0): there is no wake up timer; in order to wakeup, connect a GPIO to pin RST, the chip will wake up by a falling-edge on pin RST

### system_deep_sleep_set_option

Function:

Call this API before system_deep_sleep to set what the chip will do when deep-sleep wake up.

Prototype:

bool system_deep_sleep_set_option(uint8 option)

Parameter:

uint8 option :

deep_sleep_set_option(0): Radio calibration after deep-sleep wake up depends on init data byte 108.

deep_sleep_set_option(1): Radio calibration is done after deep-sleep wake up; this increases the current consumption.

deep_sleep_set_option(2): No radio calibration after deep-sleep wake up; this reduces the current consumption.

deep_sleep_set_option(4): Disable RF after deep-sleep wake up, just like modem sleep; this has the least current consumption; the device is not able to transmit or receive data after wake up.

Note:

Init data refers esp_init_data_default.bin.

Return:

true : succeed false : fail

### system_set_os_print

Function:

Turn on/off print logFunction

Prototype:

void system_set_os_print (uint8 onoff)

Parameters:

uint8 onoff

Note:

onoff==0: print function off onoff==1: print function on

Default:

print function on

Return:

null

### system_print_meminfo

Function:

Print memory information, including data/rodata/bss/heap

Prototype:

void system_print_meminfo (void)

Parameters: null

Return:

null

### system_get_free_heap_size

Function:

Get free heap size

Prototype:

uint32 system_get_free_heap_size(void)

Parameters: null

Return:

uint32 : available heap size

### system_os_task

Function:

Set up tasks

Prototype:

bool system_os_task( os_task_t	task,

uint8	prio, os_event_t	*queue, uint8	qlen

)

Parameters:

os_task_t task : task function

uint8 prio : task priority. 3 priorities are supported: 0/1/2; 0 is the lowest priority.

os_event_t *queue : message queue pointer uint8 qlen : message queue depth

Return:

true: succeed false: fail

Example:

#define SIG_RX

0

#define TEST_QUEUE_LEN 4 os_event_t *testQueue;

void test_task (os_event_t *e) { switch (e->sig) {

case SIG_RX:

os_printf(sig_rx %c/n, (char)e->par); break;

default:

break;

}

}

void task_init(void) {

testQueue=(os_event_t *)os_malloc(sizeof(os_event_t)*TEST_QUEUE_LEN); system_os_task(test_task,USER_TASK_PRIO_0,testQueue,TEST_QUEUE_LEN);

}

### system_os_post

Function: send message to task

Prototype:

bool system_os_post ( uint8 prio, os_signal_t sig, os_param_t par

)

Parameters:

uint8 prio

: task priority, corresponding to that you set up

os_signal_t sig : message type os_param_t par : message parameters

Return:

true: succeed false: fail

Referring to the above example: void task_post(void) {

system_os_post(USER_TASK_PRIO_0, SIG_RX, ‘a’);

}

Printout:

sig_rx a

### system_get_time

Function:

Get system time (us).

Prototype:

uint32 system_get_time(void)

Parameter: null

Return:

System time in microsecond.

### system_get_rtc_time

Function: Get RTC time, as denoted by the number of RTC clock periods.

Example:

If system_get_rtc_time returns 10 (it means 10 RTC cycles), and system_rtc_clock_cali_proc returns 5 (means 5us per RTC cycle), then the real time is 10 x 5 = 50 us.

Note:

System time will return to zero because of system_restart, but RTC still goes on.

Prototype:

uint32 system_get_rtc_time(void)

Parameter: null

Return:

RTC time

### system_rtc_clock_cali_proc

Function:

Get RTC clock period.

Prototype:

uint32 system_rtc_clock_cali_proc(void)

Parameter: null

Return:

RTC clock period (in us), bit11~ bit0 are decimal.

Note:

see RTC demo in Appendix.

### system_rtc_mem_write

Function:

During deep sleep, only RTC still working, so maybe we need to save some user data in RTC memory. Only user data area can be used by user.

|<--------system data--------->|<-----------------user data	>|

|	256 bytes	|	512 bytes	|

Note:

RTC memory is 4 bytes aligned for read and write operations. Parameter des_addr means block number(4 bytes per block). So, if we want to save some data at the beginning of user data area, des_addr will be 256/4 = 64, save_size will be data length.

Prototype:

bool system_rtc_mem_write ( uint32 des_addr,

void * src_addr, uint32 save_size

)

Parameter:

uint32 des_addr : des_addr >=64 void * src_addr : uint32 save_size :

destination address (block number) in RTC memory,

data pointer.

data length ( byte)

Return:

true: succeed false: fail

### system_rtc_mem_read

Function:

Read user data from RTC memory. Only user data area should be accessed by the user.

|<--------system data--------->|<-----------------user data	>|

|	256 bytes	|	512 bytes	|

Note:

RTC memory is 4 bytes aligned for read and write operations. Parameter src_addr means block number(4 bytes per block). So, to read data from the beginning of user data area, src_addr will be 256/4=64, save_size will be data length.

Prototype:

bool system_rtc_mem_read ( uint32 src_addr,

void * des_addr, uint32 save_size

)

Parameter:

uint32 src_addr : 64

void * des_addr :

uint32 save_size :

source address (block number) in rtc memory, src_addr >=

data pointer

data length, byte

Return:

true: succeed false: fail

### system_uart_swap

Function:

UART0 swap. Use MTCK as UART0 RX, MTDO as UART0 TX, so ROM log will not output from this new UART0. We also need to use MTDO (U0CTS) and MTCK (U0RTS) as UART0 in hardware.

Prototype:

void system_uart_swap (void)

Parameter: null

Return:

null

### system_get_boot_version

Function:

Get version info of boot

Prototype:

uint8 system_get_boot_version (void)

Parameter: null

Return:

Version info of boot.

Note:

If boot version >= 3 , you could enable boot enhance mode (refer to system_restart_enhance)

### system_get_userbin_addr

Function: Get address of the current running user bin (user1.bin or user2.bin).

Prototype:

uint32 system_get_userbin_addr (void)

Parameter: null

Return:

Start address info of the current running user bin.

### system_get_boot_mode

Function: Get boot mode.

Prototype:

uint8 system_get_boot_mode (void)

Parameter: null

Return:

#define SYS_BOOT_ENHANCE_MODE 0

#define SYS_BOOT_NORMAL_MODE 1

Note:

Enhance boot mode: can load and run FW at any address;

Normal boot mode: can only load and run normal user1.bin (or user2.bin).

### system_restart_enhance

Function:

Restarts system, and enters enhance boot mode.

Prototype:

bool system_restart_enhance( uint8 bin_type,

uint32 bin_addr

)

Parameter:

uint8 bin_type : type of bin

#define SYS_BOOT_NORMAL_BIN 0 // user1.bin or user2.bin

#define SYS_BOOT_TEST_BIN 1	// can only be Espressif test bin uint32 bin_addr : start address of bin file

Return:

true: succeed false: Fail

Note:

SYS_BOOT_TEST_BIN is for factory test during production; you can apply for the test bin from Espressif Systems.

### system_update_cpu_freq

Function:

Set CPU frequency. Default is 80MHz.

Prototype:

bool system_update_cpu_freq(uint8 freq)

Parameter:

uint8 freq : CPU frequency #define SYS_CPU_80MHz 80

#define SYS_CPU_160MHz 160

Return:

true: succeed false: fail

### system_get_cpu_freq

Function:

Get CPU frequency.

Prototype:

uint8 system_get_cpu_freq(void)

Parameter: null

Return:

CPU frequency, unit : MHz.

## SPI Flash Related APIs

### spi_flash_get_id

Function:

Get ID info of spi flash

Prototype:

uint32 spi_flash_get_id (void)

Parameters: null

Return:

SPI flash ID

### spi_flash_erase_sector

Function:

Erase sector in flash

Note:

More details in document Espressif IOT Flash RW Operation

Prototype:

SpiFlashOpResult spi_flash_erase_sector (uint16 sec)

Parameters:

uint16 sec : Sector number, the count starts at sector 0, 4KB per sector.

Return:

typedef enum{

SPI_FLASH_RESULT_OK, SPI_FLASH_RESULT_ERR, SPI_FLASH_RESULT_TIMEOUT

} SpiFlashOpResult;

### spi_flash_write

Function:

Write data to flash. Flash read/write has to be 4-bytes aligned.

Note:

More details in document Espressif IOT Flash RW Operation

Prototype:

SpiFlashOpResult spi_flash_write ( uint32 des_addr,

uint32 *src_addr, uint32 size

)

Parameters:

uint32 des_addr : destination address in flash. uint32 *src_addr : source address of the data. uint32 size	:length of data

Return:

typedef enum{

SPI_FLASH_RESULT_OK, SPI_FLASH_RESULT_ERR, SPI_FLASH_RESULT_TIMEOUT

} SpiFlashOpResult;

### spi_flash_read

Function:

Read data from flash. Flash read/write has to be 4-bytes aligned.

Prototype:

SpiFlashOpResult spi_flash_read( uint32 src_addr,

uint32 * des_addr, uint32 size

)

Parameters:

uint32 src_addr: source address in flash

uint32 *des_addr: destination address to keep data. uint32 size:	length of data

Return:

typedef enum { SPI_FLASH_RESULT_OK, SPI_FLASH_RESULT_ERR, SPI_FLASH_RESULT_TIMEOUT

} SpiFlashOpResult;

Example：

uint32 value;

uint8 *addr = (uint8 *)&value;

spi_flash_read(0x3E * SPI_FLASH_SEC_SIZE, (uint32 *)addr, 4);

os_printf("0x3E sec:%02x%02x%02x%02x\r\n", addr[0], addr[1], addr[2], addr[3]);

## WIFI Related APIs

Flash system parameter area is the last 16KB of flash.

### wifi_get_opmode

Function:

get WiFi current operating mode

Prototype:

uint8 wifi_get_opmode (void)

Parameters: null

Return:

WiFi working modes: 0x01: station mode 0x02: soft-AP mode

0x03: station+soft-AP

### wifi_get_opmode_default

Function:

get WiFi operating mode that saved in flash

Prototype:

uint8 wifi_get_opmode_default (void)

Parameters: null

Return:

WiFi working modes: 0x01: station mode 0x02: soft-AP mode

0x03: station+soft-AP

### wifi_set_opmode

Function:

Sets WiFi working mode as station, soft-AP or station+soft-AP, and save it to flash. Default is soft-AP mode.

Note:

Versions before esp_iot_sdk_v0.9.2, need to call system_restart() after this api; after esp_iot_sdk_v0.9.2, need not to restart.

This configuration will be saved in flash system parameter area if changed.

Prototype:

bool wifi_set_opmode (uint8 opmode)

Parameters:

uint8 opmode: WiFi operating modes: 0x01: station mode

0x02: soft-AP mode 0x03: station+soft-AP

Return:

true: succeed false: fail

### wifi_set_opmode_current

Function:

Sets WiFi working mode as station, soft-AP or station+soft-AP, and won’t save to flash

Prototype:

bool wifi_set_opmode_current (uint8 opmode)

Parameters:

uint8 opmode: WiFi operating modes: 0x01: station mode

0x02: soft-AP mode 0x03: station+soft-AP

Return:

true: succeed false: fail

### wifi_station_get_config

Function:

Get WiFi station current configuration

Prototype:

bool wifi_station_get_config (struct station_config *config)

Parameters:

struct station_config *config : WiFi station configuration pointer

Return:

true: succeed false: fail

### wifi_station_get_config_default

Function:

Get WiFi station configuration that saved in flash

Prototype:

bool wifi_station_get_config_default (struct station_config *config)

Parameters:

struct station_config *config : WiFi station configuration pointer

Return:

true: succeed false: fail

### wifi_station_set_config

Function:

Set WiFi station configuration, and save it to flash

Note:

If wifi_station_set_config is called in user_init , there is no need to call wifi_station_connect after that, ESP8266 will connect to router automatically; otherwise, need wifi_station_connect to connect.

In general, station_config.bssid_set need to be 0, otherwise it will check bssid which is the MAC address of AP.

This configuration will be saved in flash system parameter area if changed.

Prototype:

bool wifi_station_set_config (struct station_config *config)

Parameters:

struct station_config *config: WiFi station configuration pointer

Return:

true: succeed false: fail

### wifi_station_set_config_current

Function:

Set WiFi station configuration, won’t save to flash

Note:

If wifi_station_set_config_current is called in user_init , there is no need to call wifi_station_connect after that, ESP8266 will connect to router automatically; otherwise, need wifi_station_connect to connect.

In general, station_config.bssid_set need to be 0, otherwise it will check bssid which is the MAC address of AP.

Prototype:

bool wifi_station_set_config_current (struct station_config *config)

Parameters:

struct station_config *config: WiFi station configuration pointer

Return:

true: succeed false: fail

### wifi_station_connect

Function:

To connect WiFi station to AP

Note:

If ESP8266 has already connected to a router, then we need to call wifi_station_disconnect first, before calling wifi_station_connect.

Prototype:

bool wifi_station_connect (void)

Parameters: null

Return:

true: succeed false: fail

### wifi_station_disconnect

Function:

Disconnects WiFi station from AP

Prototype:

bool wifi_station_disconnect (void)

Parameters: null

Return:

true: succeed false: fail

### wifi_station_get_connect_status

Function:

Get connection status of WiFi station to AP

Prototype:

uint8 wifi_station_get_connect_status (void)

Parameters: null

Return:

enum{

STATION_IDLE = 0, STATION_CONNECTING, STATION_WRONG_PASSWORD, STATION_NO_AP_FOUND, STATION_CONNECT_FAIL, STATION_GOT_IP

};

### wifi_station_scan

Function:

Scan all available APs

Note:

Do not call this API in user_init. This API need to be called after system initialize done and station enable.

Prototype:

bool wifi_station_scan (struct scan_config *config, scan_done_cb_t cb);

Structure:

struct scan_config {

uint8 *ssid;	// AP’s ssid uint8 *bssid;	// AP’s bssid

uint8 channel;	//scan a specific channel

uint8 show_hidden; //scan APs of which ssid is hidden.

};

Parameters:

struct scan_config *config: AP config for scan if config==null: scan all APs

if config.ssid==null && config.bssid==null && config.channel!=null: ESP8266 will scan the specific channel.

scan_done_cb_t cb: callback function after scan

Return:

true: succeed false: fail

### scan_done_cb_t

Function:

Callback function for wifi_station_scan

Prototype:

void scan_done_cb_t (void *arg, STATUS status)

Parameters:

void *arg: information of APs that be found, refer to struct bss_info STATUS status: get status

Return:

null

Example:

wifi_station_scan(&config, scan_done);

static void ICACHE_FLASH_ATTR scan_done(void *arg, STATUS status) { if (status == OK) {

struct bss_info *bss_link = (struct bss_info *)arg; bss_link = bss_link->next.stqe_next; //ignore first

...

}

}

### wifi_station_ap_number_set

Function:

Sets the number of APs that will be cached for ESP8266 station mode. Whenever ESP8266 station connects to an AP, it keeps caches a record of this AP's SSID and password. The cached ID index starts from 0.

This configuration will be saved in flash system parameter area if changed.

Prototype:

bool wifi_station_ap_number_set (uint8 ap_number)

Parameters:

uint8 ap_number: the number of APs can be recorded (MAX: 5)

Return:

true: succeed false: fail

### wifi_station_get_ap_info

Function:

Get information of APs recorded by ESP8266 station.

Prototype:

uint8 wifi_station_get_ap_info(struct station_config config[])

Parameters:

struct station_config config[]: information of APs, array size has to be 5.

Return:

The number of APs recorded.

Example:

struct station_config config[5];

int i = wifi_station_get_ap_info(config);

### wifi_station_ap_change

Function:

Switch ESP8266 station connection to AP as specified

Prototype:

bool wifi_station_ap_change (uint8 new_ap_id)

Parameters:

uint8 new_ap_id : AP’s record id, start counting from 0.

Return:

true: succeed false: fail

### wifi_station_get_current_ap_id

Function:

Get the current record id of AP.

Prototype:

uint8 wifi_station_get_current_ap_id ();

Parameter: null

Return:

The index of the AP, which ESP8266 is currently connected to, in the cached AP list.

### wifi_station_get_auto_connect

Function:

Checks if ESP8266 station mode will connect to AP (which is cached) automatically or not when it is powered on.

Prototype:

uint8 wifi_station_get_auto_connect(void)

Parameter: null

Return:

0:	wil not connect to AP automatically; Non-0: will connect to AP automatically.

### wifi_station_set_auto_connect

Function:

Set whether ESP8266 station will connect to AP (which is recorded) automatically or not when power on. Default to enable auto-connect.

Note:

Call this API in user_init, it is effective in this current power on; call it in other place, it will be effective in next power on.

This configuration will be saved in flash system parameter area if changed.

Prototype:

bool wifi_station_set_auto_connect(uint8 set)

Parameter:

uint8 set: Automatically connect or not: 0: will not connect automatically 1: to connect automatically

Return:

true: succeed false: fail

### wifi_station_dhcpc_start

Function:

Enable ESP8266 station DHCP client.

Note:

DHCP default enable.

This configuration interacts with static IP API (wifi_set_ip_info): If enable DHCP, static IP will be disabled;

If enable static IP, DHCP will be disabled; This will depend on the last configuration.

Prototype:

bool wifi_station_dhcpc_start(void)

Parameter: null

Return:

true: succeed false: fail

### wifi_station_dhcpc_stop

Function:

Disable ESP8266 station DHCP client.

Note:

DHCP default enable.

Prototype:

bool wifi_station_dhcpc_stop(void)

Parameter: null

Return:

true: succeed false: fail

### wifi_station_dhcpc_status

Function: Get ESP8266 station DHCP client status.

Prototype:

enum dhcp_status wifi_station_dhcpc_status(void)

Parameter: null

Return:

enum dhcp_status { DHCP_STOPPED, DHCP_STARTED

};

### wifi_station_set_reconnect_policy

Function:

Set whether reconnect or not when ESP8266 station disconnected from AP

Note：

We suggest to call this API in user_init

This API can only be called when ESP8266 station enable.

Prototype:

bool wifi_station_set_reconnect_policy(bool set)

Parameter:

bool set - true, enable reconnect；false，disable reconnect

Return:

true：succeed false：fail

### wifi_softap_get_config

Function:

Get WiFi soft-AP current configuration

Prototype:

bool wifi_softap_get_config(struct softap_config *config)

Parameter:

struct softap_config *config : ESP8266 soft-AP config

Return:

true: succeed false: fail

### wifi_softap_get_config_default

Function:

Get WiFi soft-AP configuration that saved in flash

Prototype:

bool wifi_softap_get_config_default(struct softap_config *config)

Parameter:

struct softap_config *config : ESP8266 soft-AP config

Return:

true: succeed false: fail

### wifi_softap_set_config

Function:

Set WiFi soft-AP configuration and save it to flash

Note：

- This configuration will be saved in flash system parameter area if changed.

- In soft-AP + station mode, ESP8266 soft-AP will adjust its channel configuration to be as same as ESP8266. More details in appendix or BBS http://bbs.espressif.com/viewtopic.php?f=10&t=324

Prototype:

bool wifi_softap_set_config (struct softap_config *config)

Parameter:

struct softap_config *config : WiFi soft-AP configuration pointer

Return:

true: succeed false: fail

### wifi_softap_set_config_current

Function:

Set WiFi soft-AP configuration, won’t save it to flash

Note:

In soft-AP + station mode, ESP8266 soft-AP will adjust its channel configuration to be as same as ESP8266. More details in appendix or BBS http://bbs.espressif.com/viewtopic.php?f=10&t=324

Prototype:

bool wifi_softap_set_config_current (struct softap_config *config)

Parameter:

struct softap_config *config : WiFi soft-AP configuration pointer

Return:

true: succeed false: fail

### wifi_softap_get_station_info

Function:

Get connected station devices under soft-AP mode, including MAC and IP

Note:

This API can only be used when soft-AP DHCP enabled.

Prototype:

struct station_info * wifi_softap_get_station_info(void)

Input Parameters: null

Return:

struct station_info* : station information structure

### wifi_softap_free_station_info

Function:

Frees the struct station_info by calling the wifi_softap_get_station_info function

Prototype:

void wifi_softap_free_station_info(void)

Input Parameters: null

Return:

null

Examples 1 (Getting MAC and IP information):

struct station_info * station = wifi_softap_get_station_info(); struct station_info * next_station;

while(station) {

os_printf(bssid : MACSTR, ip : IPSTR/n,

MAC2STR(station->bssid), IP2STR(&station->ip)); next_station = STAILQ_NEXT(station, next); os_free(station);	// Free it directly

station = next_station;

}

Examples 2 (Getting MAC and IP information):

struct station_info * station = wifi_softap_get_station_info(); while(station){

os_printf(bssid : MACSTR, ip : IPSTR/n,

MAC2STR(station->bssid), IP2STR(&station->ip)); station = STAILQ_NEXT(station, next);

}

wifi_softap_free_station_info();	// Free it by calling functions

### wifi_softap_dhcps_start

Function: Enable ESP8266 soft-AP DHCP server.

Note:

DHCP default enable.

This configuration interacts with static IP API (wifi_set_ip_info): If enable DHCP, static IP will be disabled;

If enable static IP, DHCP will be disabled; This will depend on the last configuration.

Prototype:

bool wifi_softap_dhcps_start(void)

Parameter: null

Return:

true: succeed false: fail

### wifi_softap_dhcps_stop

Function: Disable ESP8266 soft-AP DHCP server.

Note: DHCP default enable.

Prototype:

bool wifi_softap_dhcps_stop(void)

Parameter: null

Return:

true: succeed false: fail

### wifi_softap_set_dhcps_lease

Function:

Set the IP range that can be got from ESP8266 soft-AP DHCP server.

Note:

This API has to be called during DHCP server disable(wifi_softap_dhcps_stop)

This configuration only take effect on next wifi_softap_dhcps_start, if then wifi_softap_dhcps_stop is called； user needs to call this API to set IP range again if needed，then call wifi_softap_dhcps_start to take effect.

Prototype:

bool wifi_softap_set_dhcps_lease(struct dhcps_lease *please)

Parameter:

struct dhcps_lease {

struct ip_addr start_ip; struct ip_addr end_ip;

};

Return:

true: succeed false: fail

Example:

void dhcps_lease_test(void)

{

struct dhcps_lease dhcp_lease;

const char* start_ip = “192.168.5.100”; const char* end_ip = “192.168.5.105”;

dhcp_lease.start_ip.addr = ipaddr_addr(start_ip); dhcp_lease.end_ip.addr = ipaddr_addr(end_ip); wifi_softap_set_dhcps_lease(&dhcp_lease);

}

or

void dhcps_lease_test(void)

{

struct dhcps_lease dhcp_lease; IP4_ADDR(&dhcp_lease.start_ip, 192, 168, 5, 100);

IP4_ADDR(&dhcp_lease.end_ip, 192, 168, 5, 105); wifi_softap_set_dhcps_lease(&dhcp_lease);

}

### wifi_softap_dhcps_status

Function: Get ESP8266 soft-AP DHCP server status.

Prototype:

enum dhcp_status wifi_softap_dhcps_status(void)

Parameter: null

Return:

enum dhcp_status { DHCP_STOPPED, DHCP_STARTED

};

### wifi_softap_dhcps_set_offer_option

Function:

Set ESP8266 soft-AP DHCP server option.

Structure：

enum dhcps_offer_option{ OFFER_START = 0x00, OFFER_ROUTER = 0x01,

OFFER_END

};

Prototype:

bool wifi_softap_dhcps_set_offer_option(uint8 level, void* optarg)

Parameter:

uint8 level -	OFFER_ROUTER set router option void* optarg -	default to be enable

bit0, 0 disable router information from ESP8266 softAP DHCP server；

bit0, 1 enable router information from ESP8266 softAP DHCP server；

Return:

true : succeed false : fail

Example:

uint8 mode = 0； wifi_softap_dhcps_set_offer_option(OFFER_ROUTER, &mode);

### wifi_set_phy_mode

Fuction: Set ESP8266 physical mode (802.11b/g/n).

Note: ESP8266 soft-AP only support bg.

Prototype:

bool wifi_set_phy_mode(enum phy_mode mode)

Parameter:

enum phy_mode mode : physical mode enum phy_mode {

PHY_MODE_11B = 1,

PHY_MODE_11G = 2,

PHY_MODE_11N = 3

};

Return:

true : succeed false : fail

### wifi_get_phy_mode

Function:

Get ESP8266 physical mode (802.11b/g/n)

Prototype:

enum phy_mode wifi_get_phy_mode(void)

Parameter: null

Return:

enum phy_mode{ PHY_MODE_11B = 1,

PHY_MODE_11G = 2,

PHY_MODE_11N = 3

};

### wifi_get_ip_info

Function:

Get IP info of WiFi station or soft-AP interface

Prototype:

bool wifi_get_ip_info( uint8 if_index, struct ip_info *info

)

Parameters:

uint8 if_index : the interface to get IP info: 0x00 for STATION_IF, 0x01 for SOFTAP_IF.

struct ip_info *info : pointer to get IP info of a certain interface

Return:

true: succeed false: fail

### wifi_set_ip_info

Function:

Set IP address of ESP8266 station or soft-AP

Note:

To set static IP, please disable DHCP first (wifi_station_dhcpc_stop or wifi_softap_dhcps_stop):

If enable static IP, DHCP will be disabled;

If enable DHCP, static IP will be disabled;

Prototype:

bool wifi_set_ip_info( uint8 if_index, struct ip_info *info

)

Prototype:

uint8 if_index : set station IP or soft-AP IP #define STATION_IF	0x00

#define SOFTAP_IF	0x01 struct ip_info *info : IP information

Example:

struct ip_info info; wifi_station_dhcpc_stop(); wifi_softap_dhcps_stop();

IP4_ADDR(&info.ip, 192, 168, 3, 200);

IP4_ADDR(&info.gw, 192, 168, 3, 1);

IP4_ADDR(&info.netmask, 255, 255, 255, 0); wifi_set_ip_info(STATION_IF, &info);

IP4_ADDR(&info.ip, 10, 10, 10, 1);

IP4_ADDR(&info.gw, 10, 10, 10, 1);

IP4_ADDR(&info.netmask, 255, 255, 255, 0); wifi_set_ip_info(SOFTAP_IF, &info);

Return:

true: succeed false: fail

### wifi_set_macaddr

Function:

Sets MAC address

Note:

Can only be used in user_init.

Prototype:

bool wifi_set_macaddr( uint8 if_index, uint8 *macaddr

)

Parameter:

uint8 if_index : set station MAC or soft-AP mac #define STATION_IF	0x00

#define SOFTAP_IF	0x01 uint8 *macaddr : MAC address

Example:

char sofap_mac[6] = {0x16, 0x34, 0x56, 0x78, 0x90, 0xab}; char sta_mac[6] = {0x12, 0x34, 0x56, 0x78, 0x90, 0xab}; wifi_set_macaddr(SOFTAP_IF, sofap_mac); wifi_set_macaddr(STATION_IF, sta_mac);

Return:

true: succeed false: fail

### wifi_get_macaddr

Function: get MAC address

Prototype:

bool wifi_get_macaddr( uint8 if_index, uint8 *macaddr

)

Parameter:

uint8 if_index : set station MAC or soft-AP mac #define STATION_IF	0x00

#define SOFTAP_IF	0x01 uint8 *macaddr : MAC address

Return:

true: succeed false: fail

### wifi_set_sleep_type

Function:

Sets sleep type for power saving. Set NONE_SLEEP_T to disable power saving.

Note: Default to be Modem sleep.

Prototype:

bool wifi_set_sleep_type(enum sleep_type type)

Parameters:

enum sleep_type type : sleep type

Return:

true: succeed false: fail

### wifi_get_sleep_type

Function:

Gets sleep type.

Prototype:

enum sleep_type wifi_get_sleep_type(void)

Parameters: null

Return:

enum sleep_type { NONE_SLEEP_T = 0; LIGHT_SLEEP_T, MODEM_SLEEP_T

};

### wifi_status_led_install

Function:

Installs WiFi status LED

Prototype:

void wifi_status_led_install ( uint8 gpio_id,

uint32 gpio_name, uint8 gpio_func

)

Parameter:

uint8 gpio_id	: gpio number uint8 gpio_name : gpio mux name uint8 gpio_func : gpio function

Return:

null

Example:

Use GPIO0 as WiFi status LED #define HUMITURE_WIFI_LED_IO_MUX #define HUMITURE_WIFI_LED_IO_NUM #define HUMITURE_WIFI_LED_IO_FUNC

PERIPHS_IO_MUX_GPIO0_U 0

FUNC_GPIO0

wifi_status_led_install(HUMITURE_WIFI_LED_IO_NUM,

HUMITURE_WIFI_LED_IO_MUX, HUMITURE_WIFI_LED_IO_FUNC)

### wifi_status_led_uninstall

Function: Uninstall WiFi status LED

Prototype:

void wifi_status_led_uninstall ()

Parameter: null

Return:

null

### wifi_set_broadcast_if

Function:

Set ESP8266 send UDP broadcast from station interface or soft-AP interface, or both station and soft-AP interfaces. Default to be soft-AP.

Note：

If set broadcast interface to be station only，ESP8266 softAP DHCP server will be disable.

Prototype:

bool wifi_set_broadcast_if (uint8 interface)

Parameter:

uint8 interface : 1:station; 2:soft-AP, 3:station+soft-AP

Return:

true: succeed false: fail

### wifi_get_broadcast _if

Function:

Get interface which ESP8266 sent UDP broadcast from. This is usually used when you have STA+soft-AP mode to avoid ambiguity.

Prototype:

uint8 wifi_get_broadcast_if (void)

Parameter: null

Return:

1: station

2: soft-AP

3: both station and soft-AP

### wifi_set_event_handler_cb

Function:

Register WiFi event handler

Prototype:

void wifi_set_event_handler_cb(wifi_event_handler_cb_t cb)

Parameter:

wifi_event_handler_cb_t cb - callback

返回:

无

示例：

void wifi_handle_event_cb(System_Event_t *evt)

{

os_printf("event %x\n", evt->event); switch (evt->event) {

case EVENT_STAMODE_CONNECTED:

os_printf("connect to ssid %s, channel %d\n", evt->event_info.connected.ssid,

evt->event_info.connected.channel);

break;

case EVENT_STAMODE_DISCONNECTED:

os_printf("disconnect from ssid %s, reason %d\n",

evt->event_info.disconnected.ssid, evt->event_info.disconnected.reason);

break;

case EVENT_STAMODE_AUTHMODE_CHANGE:

os_printf("mode: %d -> %d\n",

evt->event_info.auth_change.old_mode, evt->event_info.auth_change.new_mode);

break;

case EVENT_STAMODE_GOT_IP:

os_printf("ip:" IPSTR ",mask:" IPSTR ",gw:" IPSTR,

IP2STR(&evt->event_info.got_ip.ip), IP2STR(&evt->event_info.got_ip.mask), IP2STR(&evt->event_info.got_ip.gw));

os_printf("\n"); break;

case EVENT_SOFTAPMODE_STACONNECTED:

os_printf("station: " MACSTR "join, AID = %d\n", MAC2STR(evt->event_info.sta_connected.mac), evt->event_info.sta_connected.aid);

break;

case EVENT_SOFTAPMODE_STADISCONNECTED:

os_printf("station: " MACSTR "leave, AID = %d\n", MAC2STR(evt->event_info.sta_disconnected.mac), evt->event_info.sta_disconnected.aid);

break; default:

break;

}

}

void user_init(void)

{

// TODO: add your own code here.... wifi_set_event_hander_cb(wifi_handle_event_cb);

}

## Upgrade (FOTA) APIs

### system_upgrade_userbin_check

Function:

Checks user bin

Prototype:

uint8 system_upgrade_userbin_check()

Input Parameters: none

Return:

0x00 : UPGRADE_FW_BIN1, i.e. user1.bin

0x01 : UPGRADE_FW_BIN2, i.e. user2.bin

### system_upgrade_flag_set

Function:

Sets upgrade status flag.

Note:

If you using system_upgrade_start to upgrade, this API need not be called. If you using spi_flash_write to upgrade firmware yourself, this flag need to be set to UPGRADE_FLAG_FINISH, then call system_upgrade_reboot to reboot to run new firmware.

Prototype:

void system_upgrade_flag_set(uint8 flag)

Parameter:

uint8 flag:

#define UPGRADE_FLAG_IDLE #define UPGRADE_FLAG_START #define UPGRADE_FLAG_FINISH

0x00

0x01

0x02

Return:

null

### system_upgrade_flag_check

Function:

Gets upgrade status flag.

Prototype:

uint8 system_upgrade_flag_check()

Parameter:

null

Return:

#define UPGRADE_FLAG_IDLE #define UPGRADE_FLAG_START #define UPGRADE_FLAG_FINISH

0x00

0x01

0x02

### system_upgrade_start

Function:

Configures parameters and start upgrade

Prototype:

bool system_upgrade_start (struct upgrade_server_info *server)

Parameters:

struct upgrade_server_info *server : server related parameters

Return:

true: start upgrade

false: upgrade can’t be started.

### system_upgrade_reboot

Function: reboot system and use new version

Prototype:

void system_upgrade_reboot (void)

Parameters: none

Return:

none

## Sniffer Related APIs

### wifi_promiscuous_enable

Function:

Enable promiscuous mode for sniffer

Note：

- promiscuous mode can only be enabled in station mode.

- During promiscuous mode（sniffer），ESP8266 station and soft-AP are disabled.

- Before enable promiscuous mode，please disable auto-connection function first wifi_station_set_auto_connect(0)

- Don’t call any other APIs during sniffer，please call wifi_promiscuous_enable(0) first.

Prototype:

void wifi_promiscuous_enable(uint8 promiscuous)

Parameter:

uint8 promiscuous :

0: disable promiscuous;

1: enable promiscuous

Return:

null

### wifi_promiscuous_set_mac

Function:

Set MAC address filter for sniffer.

Note:

This filter only available in the current sniffer phase, if you disable sniffer and then enable sniffer, you need to set filter again if you need it.

Prototype:

void wifi_promiscuous_set_mac(const uint8_t *address)

Parameter:

const uint8_t *address : MAC address

Return:

null

### wifi_set_promiscuous_rx_cb

Function:

Registers an RX callback function in promiscuous mode, which will be called when data packet is received.

Prototype:

void wifi_set_promiscuous_rx_cb(wifi_promiscuous_cb_t cb)

Parameter:

wifi_promiscuous_cb_t cb : callback

Return:

null

### wifi_get_channel

Function:

Get channel number for sniffer functions

Prototype:

uint8 wifi_get_channel(void)

Parameters: null

Return:

Channel number

### wifi_set_channel

Function:

Set channel number for sniffer functions

Prototype:

bool wifi_set_channel (uint8 channel)

Parameters:

uint8 channel : channel number

Return:

true: succeed false: fail

## smart config APIs

### smartconfig_start

#### Function:

Start smart configuration mode, to connect ESP8266 station to AP, by sniffing for special packets from the air, containing SSID and password of desired AP. You need to broadcast the SSID and password (e.g. from mobile device or computer) with the SSID and password encoded.

#### Note:

      - This api can only be called in station mode.

      - During smart config，ESP8266 station and soft-AP are disabled.

      - Can not call smartconfig_start twice before it finish，please call smartconfig_stop first.

      - Don’t call any other APIs during smart config, please call smartconfig_stop first.

#### Prototype:

bool smartconfig_start( sc_type type, sc_callback_t cb, uint8 log

)

#### Parameter:

sc_type type : smart config protocol type: AirKiss or ESP-TOUCH. sc_callback_t cb : pointer to struct station_config; smart config callback; executed when ESP8266 successfully obtains SSID and password of target AP; for parameter of this callback, more information in example.

uint8 log : 1: UART output logs; otherwise: UART only outputs the result.

#### Return:

true: succeed false: fail

Example:

void ICACHE_FLASH_ATTR

smartconfig_done(void *data) {

struct station_config *sta_conf = data; wifi_station_set_config(sta_conf); wifi_station_disconnect(); wifi_station_connect(); user_devicefind_init(); user_esp_platform_init();

}

smartconfig_start(SC_TYPE_ESPTOUCH,smartconfig_done);

### smartconfig_stop

Function:

stop smart config, free the buffer taken by smartconfig_start.

Note:

Whether connect to AP succeed or not, this API should be called to free memory taken by smartconfig_start.

Prototype:

bool smartconfig_stop(void)

Parameter: null

Return:

true: succeed false: fail

### get_smartconfig_status

Function:

Get smart config status

Note:

Can not call this API after smartconfig_stop, because smartconfig_stop will try to free (again) the memory which contains this smart config status.

Prototype:

sc_status get_smartconfig_status(void)

Parameter: null

Return:

typedef enum { SC_STATUS_WAIT = 0, SC_STATUS_FIND_CHANNEL,

SC_STATUS_GETTING_SSID_PSWD, SC_STATUS_GOT_SSID_PSWD, SC_STATUS_LINK, SC_STATUS_LINK_OVER,

} sc_status;

Note:

Use APP to start connection when get_smartconfig_status is SC_STATUS_FIND_CHANNEL.

## SNTP APIs

### sntp_setserver

Function:

Set SNTP server by IP address, support 3 SNTP server at most

Prototype:

void sntp_setserver(unsigned char idx, ip_addr_t *addr)

Parameter:

unsigned char idx : SNTP server index， support 3 SNTP server at most（0 ~ 2）；index 0 is the main server，index 1 and 2 are as backup.

ip_addr_t *addr : IP address; users need to ensure that it’s a SNTP server

Return:

none

### sntp_getserver

Function:

Get IP address of SNTP server which set by sntp_setserver

Prototype:

ip_addr_t sntp_getserver(unsigned char idx)

Parameter:

unsigned char idx : SNTP server index, support 3 SNTP server at most（0 ~ 2）

Return:

IP address

### sntp_setservername

Function:

Set SNTP server by domain name, support 3 SNTP server at most

Prototype:

void sntp_setservername(unsigned char idx, char *server)

Parameter:

unsigned char idx : SNTP server index， support 3 SNTP server at most（0 ~ 2）；index 0 is the main server，index 1 and 2 are as backup.

char *server : domain name； users need to ensure that it’s a SNTP server

Return:

none

### sntp_getservername

Function:

Get domain name of SNTP server which set by sntp_setservername

Prototype:

char * sntp_getservername(unsigned char idx)

Parameter:

unsigned char idx : SNTP server index， support 3 SNTP server at most（0 ~ 2)

Return:

domain name

### sntp_init

Function:

SNTP initialize

Prototype:

void sntp_init(void)

Parameter: none

Return:

none

### sntp_stop

Function:

Stop SNTP

Prototype:

void sntp_stop(void)

Parameter: none

Return:

none

### sntp_get_current_timestamp

Function:

Get current timestamp from basic time（1970.01.01 00：00：00 GMT）, uint:second

Prototype:

uint32 sntp_get_current_timestamp()

Parameter: none

Return:

time stamp

### sntp_get_real_time

Function:

Get real time（GMT + 8）

Prototype:

char* sntp_get_real_time(long t)

Parameter:

long t - time stamp

Return:

real time

### SNTP Example

ip_addr_t *addr = (ip_addr_t *)os_zalloc(sizeof(ip_addr_t)); sntp_setservername(0, ”us.pool.ntp.org”); // set server 0 by domain name

sntp_setservername(1, ”ntp.sjtu.edu.cn”); // set server 1 by domain name ipaddr_aton(“210.72.145.44”, addr);

sntp_setserver(2, addr); // set server 2 by IP address sntp_init();

os_free(addr);

uint32 current_stamp;

current_stamp = sntp_get_current_timestamp();

os_printf(“sntp: %d, %s \n”,current_stamp, sntp_get_real_time(current_stamp));

# TCP/UDP APIs

Found in esp_iot_sdk/include/espconn.h. The network APIs can be grouped into the following types:

- General APIs: APIs can be used for both TCP and UDP .

- TCP APIs: APIs that are only used for TCP.

- UDP APIs: APIs that are only used for UDP.

- mDNS APIs：APIs that related to mDNS

## Generic TCP/UDP APIs

### espconn_delete

Function:

Delete a transmission.

Note:

Corresponding creation API : TCP: espconn_accept, UDP: espconn_create

Prototype:

sint8 espconn_delete(struct espconn *espconn)

Parameter:

struct espconn *espconn : corresponding connected control block structure

Return:

0	: succeed

Non-0 : error, return error code ESPCONN_ARG - illegal argument，can’t find network transmission according to structure espconn

### espconn_gethostbyname

Function: DNS

Prototype:

err_t espconn_gethostbyname( struct espconn *pespconn, const char *hostname, ip_addr_t *addr, dns_found_callback found

)

Parameters:

struct espconn *espconn : corresponding connected control block structure const char *hostname	: domain name string pointer

ip_addr_t *addr	: IP address dns_found_callback found : callback

Return:

err_t : ESPCONN_OK - succeed

ESPCONN_INPROGRESS - error code : already connected ESPCONN_ARG - error code : illegal argument，can’t find network

transmission according to structure espconn

Example as follows. Pls refer to source code of IoT_Demo:

ip_addr_t esp_server_ip;

LOCAL void ICACHE_FLASH_ATTR

user_esp_platform_dns_found(const char *name, ip_addr_t *ipaddr, void *arg)

{

struct espconn *pespconn = (struct espconn *)arg; os_printf(user_esp_platform_dns_found %d.%d.%d.%d/n,

*((uint8 *)&ipaddr->addr), *((uint8 *)&ipaddr->addr + 1),

*((uint8 *)&ipaddr->addr + 2), *((uint8 *)&ipaddr->addr + 3));

}

void dns_test(void) { espconn_gethostbyname(pespconn,iot.espressif.cn, &esp_server_ip,

user_esp_platform_dns_found);

}

### espconn_port

Function: get void ports

Prototype:

uint32 espconn_port(void)

Parameter: null

Return:

uint32 : id of the port you get

### espconn_regist_sentcb

Function:

Register data sent function which will be called back when data are successfully sent.

Prototype:

sint8 espconn_regist_sentcb( struct espconn *espconn, espconn_sent_callback sent_cb

)

Parameters:

struct espconn *espconn : corresponding connected control block structure espconn_sent_callback sent_cb : registered callback function

Return:

0	: succeed

Non-0 : error code ESPCONN_ARG - illegal argument，can’t find network transmission according to structure espconn

### espconn_regist_recvcb

Function:

register data receive function which will be called back when data are received

Prototype:

sint8 espconn_regist_recvcb( struct espconn *espconn, espconn_recv_callback recv_cb

)

Parameters:

struct espconn *espconn : corresponding connected control block structure espconn_connect_callback connect_cb : registered callback function

Return:

0	: succeed

Non-0 : error code ESPCONN_ARG - illegal argument，can’t find network transmission according to structure espconn

### espconn_sent_callback

Function:

Callback after the data are sent

Prototype:

void espconn_sent_callback (void *arg)

Parameters:

void *arg : call back function parameters

Return:

null

### espconn_recv_callback

Function:

callback after data are received

Prototype:

void espconn_recv_callback ( void *arg,

char *pdata, unsigned short len

)

Parameters:

void *arg : callback function parameters char *pdata : received data entry parameters unsigned short len : received data length

Return:

null

### espconn_sent

Function:

Send data through WiFi

Note:

Please call espconn_sent after espconn_sent_callback of the pre-packet.

Prototype:

sint8 espconn_sent(

struct espconn *espconn, uint8 *psent,

uint16 length

)

Parameters:

struct espconn *espconn : corresponding connected control block structure uint8 *psent	: sent data pointer

uint16 length : sent data length

Return:

0	: succeed Non-0 : error code

ESPCONN_MEM - Out of memory

ESPCONN_ARG - illegal argument，can’t find network transmission according to structure espconn

## TCP APIs

TCP APIs act only on TCP connections and do not affect nor apply to UDP connections.

### espconn_accept

Function:

Creates a TCP server (i.e. accepts connections.)

Prototype:

sint8 espconn_accept(struct espconn *espconn)

Parameter:

struct espconn *espconn : corresponding connected control block structure

Return:

0	: succeed Non-0 : error code

ESPCONN_MEM - Out of memory ESPCONN_ISCONN - Already connected

ESPCONN_ARG - illegal argument，can’t find TCP connection according to

structure espconn

### espconn_secure_accept

Function:

Creates an SSL TCP server.

Note:

(1) Only created one SSL server is allowed，this API can be called only once， and only one SSL client is allowed to connect.

- If SSL encrypted packet size is larger than ESP8266 SSL buffer size (default 2KB, set by espconn_secure_set_size), SSL connection will fail, will enter espconn_reconnect_callback

- SSL related APIs named as espconn_secure_XXX are different from normal TCP APIs, so please don’t mixed use. In SSL connection, only espconn_secure_XXX APIs, espconn_regist_XXX APIs and espconn_port can be used.

Prototype:

sint8 espconn_secure_accept(struct espconn *espconn)

Parameter:

struct espconn *espconn : corresponding connected control block structure

Return:

0	: succeed Non-0 : error code

ESPCONN_MEM - Out of memory ESPCONN_ISCONN - Already connected

ESPCONN_ARG - illegal argument，can’t find TCP connection according to

structure espconn

### espconn_regist_time

Function:

register timeout interval of ESP8266 TCP server.

Note:

Call this API after espconn_accept.

If timeout is set to 0, timeout will be disable and ESP8266 TCP server will not disconnect TCP clients has stopped communication. This usage of timeout=0, is deprecated.

Prototype:

sint8 espconn_regist_time(

struct espconn *espconn, uint32 interval,

uint8 type_flag

)

Parameters:

struct espconn *espconn : corresponding connected control block structure uint32 interval : timeout interval, unit: second, maximum: 7200 seconds uint8 type_flag : 0, set all connections; 1, set a single connection

Return:

0	: succeed

Non-0 : error code ESPCONN_ARG - illegal argument，can’t find TCP connection according to structure espconn

### espconn_get_connection_info

Function:

Get a connection’s info in TCP multi-connection case

Prototype:

sint8 espconn_get_connection_info( struct espconn *espconn, remot_info **pcon_info, uint8 typeflags

)

Parameters:

struct espconn *espconn : corresponding connected control block structure remot_info **pcon_info : connect to client info

uint8 typeflags	: 0, regular server;1, ssl server

Return:

0	: succeed

Non-0 : error code ESPCONN_ARG - illegal argument，can’t find TCP connection according to structure espconn

### espconn_connect

Function:

Connect to a TCP server (ESP8266 acting as TCP client).

Prototype:

sint8 espconn_connect(struct espconn *espconn)

Parameters:

struct espconn *espconn : corresponding connected control block structure

Return:

0	: succeed Non-0 : error code

ESPCONN_RTE - Routing Problem ESPCONN_MEM - Out of memory ESPCONN_ISCONN - Already connected

ESPCONN_ARG - illegal argument，can’t find TCP connection according to structure espconn

### espconn_connect_callback

Function: successful listening (ESP8266 as TCP server) or connection (ESP8266 as TCP client) callback, register by espconn_regist_connectcb

Prototype:

void espconn_connect_callback (void *arg)

Parameter:

void *arg : callback function parameters

Return:

null

### espconn_set_opt

Function: Set option of TCP connection

Prototype:

sint8 espconn_set_opt(

struct espconn *espconn, uint8 opt

)

Structure：

enum espconn_option{

ESPCONN_START = 0x00， ESPCONN_REUSEADDR = 0x01， ESPCONN_NODELAY = 0x02， ESPCONN_COPY = 0x04， ESPCONN_KEEPALIVE = 0x08， ESPCONN_END

}

Parameter:

struct espconn *espconn : corresponding connected control structure uint8 opt : Option of TCP connection, refer to espconn_option

bit 0: 1: free memory after TCP disconnection happen need not wait 2 minutes;

bit 1: 1: disable nalgo algorithm during TCP data transmission, quiken the data transmission.

bit 2: 1: use 2920 bytes write buffer for the data espconn_sent sending.

bit 3: 1: enable TCP keep alive

Return:

0	: succeed

Non-0 : error code ESPCONN_ARG - illegal argument，can’t find TCP connection according to structure espconn

Note:

In general, we need not call this API;

If call espconn_set_opt, please call it in espconn_connect_callback.

### espconn_clear_opt

Function:

Clear option of TCP connection.

Prototype:

sint8 espconn_clear_opt(

struct espconn *espconn, uint8 opt

)

Structure：

enum espconn_option{

ESPCONN_START = 0x00， ESPCONN_REUSEADDR = 0x01， ESPCONN_NODELAY = 0x02， ESPCONN_COPY = 0x04， ESPCONN_KEEPALIVE = 0x08， ESPCONN_END

}

Parameters:

struct espconn *espconn : corresponding connected control block structure uint8 opt : option of TCP connection,refer to espconn_option

Return: 0

Non-0

: succeed

: error code ESPCONN_ARG - illegal argument，can’t find TCP

connection according to structure espconn

### espconn_set_keepalive

Function:

Set configuration of TCP keep alive .

Prototype:

sint8 espconn_set_keepalive(struct espconn *espconn, uint8 level, void* optarg)

#### Structure：

enum espconn_level{ ESPCONN_KEEPIDLE, ESPCONN_KEEPINTVL, ESPCONN_KEEPCNT

}

#### Parameters:

struct espconn *espconn : corresponding connected control block structure

uint8 level : Default to do TCP keep-alive detection every ESPCONN_KEEPIDLE, if there in no response, retry ESPCONN_KEEPCNT times every ESPCONN_KEEPINTVL. If still no response, considers it as TCP connection broke, goes into espconn_reconnect_callback .

description：

ESPCONN_KEEPIDLE - TCP keep-alive interval，unit：500 millisecond ESPCONN_KEEPINTVL - packet interval during TCP keep-alive，unit：500

millisecond

ESPCONN_KEEPCNT - maximum packet count of TCP keep-alive void* optarg ：value of parameter

#### Return:

0	: succeed

Non-0	: error code ESPCONN_ARG - illegal argument，can’t find TCP connection according to structure espconn

#### Note:

In general, we need not call this API;

If needed, please call it in espconn_connect_callback and call espconn_set_opt to enable keep alive first.

### espconn_get_keepalive

Function:

Get value of TCP keep-alive parameter

Prototype:

sint8 espconn_set_keepalive(struct espconn *espconn, uint8 level, void* optarg)

Structure：

enum espconn_level{ ESPCONN_KEEPIDLE, ESPCONN_KEEPINTVL, ESPCONN_KEEPCNT

}

Parameter:

struct espconn *espconn : corresponding connected control block structure uint8 level :

ESPCONN_KEEPIDLE - TCP keep-alive interval，unit：500 millisecond

ESPCONN_KEEPINTVL - packet interval during TCP keep-alive，unit：500 millisecond

ESPCONN_KEEPCNT - maximum packet count of TCP keep-alive

void* optarg ：value of parameter

Return: 0

Non-0

: succeed

: error code ESPCONN_ARG - illegal argument，can’t find TCP

connection according to structure espconn

### espconn_disconnect

Function:

disconnect a TCP connection

Prototype:

sint8 espconn_disconnect(struct espconn *espconn)

Parameters:

struct espconn *espconn : corresponding connected control structure

Return:

0	: succeed

Non-0 : error code ESPCONN_ARG - illegal argument，can’t find TCP connection according to structure espconn

### espconn_regist_connectcb

Function:

Register connection function which will be called back under successful TCP connection

Prototype:

sint8 espconn_regist_connectcb( struct espconn *espconn,

espconn_connect_callback connect_cb

)

Parameters:

struct espconn *espconn : corresponding connected control block structure espconn_connect_callback connect_cb : registered callback function

Return:

0	: succeed

Non-0 : error code ESPCONN_ARG - illegal argument，can’t find TCP connection according to structure espconn

### espconn_regist_reconcb

Function:

Register reconnect callback

Note:

Reconnect callback is more like a network error handler; it handles errors that occurred in any phase of the connection. For instance, if espconn_sent fails, reconnect callback will be called because the network is broken.

Prototype:

sint8 espconn_regist_reconcb( struct espconn *espconn,

espconn_connect_callback recon_cb

)

Parameters:

struct espconn *espconn : corresponding connected control block structure espconn_connect_callback connect_cb : registered callback function

Return:

0	: succeed

Non-0 : error code ESPCONN_ARG - illegal argument，can’t find TCP connection according to structure espconn

### espconn_regist_disconcb

Function: register disconnection function which will be called back under successful TCP disconnection

Prototype:

sint8 espconn_regist_disconcb( struct espconn *espconn,

espconn_connect_callback discon_cb

)

Parameters:

struct espconn *espconn : corresponding connected control block structure espconn_connect_callback connect_cb : registered callback function

Return:

0	: succeed

Non-0 : error code ESPCONN_ARG - illegal argument，can’t find TCP connection according to structure espconn

### espconn_regist_write_finish

Function:

Register a callback which will be called when all sending data is completely write into write buffer or sent.

Prototype:

sint8 espconn_regist_write_finish ( struct espconn *espconn,

espconn_connect_callback write_finish_fn

)

Parameters:

struct espconn *espconn : corresponding connected control block structure espconn_connect_callback write_finish_fn : registered callback function

Return:

0	: succeed

Non-0 : error code ESPCONN_ARG - illegal argument，can’t find TCP connection according to structure espconn

### espconn_secure_set_size

Function:

Set buffer size of encrypted data (SSL)

Note:

Buffer size default to be 2Kbytes. If need to change, please call this API before espconn_secure_accept (ESP8266 as TCP SSL server) or espconn_secure_connect (ESP8266 as TCP SSL client)

Prototype:

bool espconn_secure_set_size (uint8 level, uint16 size)

Parameters:

uint8 level : set buffer for ESP8266 SSL server/client：

0x01 SSL client；0x02 SSL server；0x03 SSL client and SSL server uint16 size : buffer size，range：1 ~ 8192，unit：byte，default to be 2048

Return:

true	: succeed false : fail

### espconn_secure_get_size

Function:

Get buffer size of encrypted data (SSL)

Prototype:

sint16 espconn_secure_get_size (uint8 level)

Parameters:

uint8 level : buffer for ESP8266 SSL server/client：

0x01 SSL client；0x02 SSL server；0x03 SSL client and SSL server

Return:

buffer size

### espconn_secure_connect

Function:

Secure connect (SSL) to a TCP server (ESP8266 is acting as TCP client.)

Note:

- Only one connection is allowed when ESP8266 as SSL client, this API can be called only once, or call espconn_secure_disconnect to disconnect first, then call this API to create another SSL connection.

- If SSL encrypted packet size is larger than ESP8266 SSL buffer size (default 2KB, set by espconn_secure_set_size), SSL connection will fail, will enter espconn_reconnect_callback

(3) SSL related APIs named as espconn_secure_XXX are different from normal TCP APIs, so please don’t mixed use. In SSL connection, only espconn_secure_XXX APIs, espconn_regist_XXX APIs and espconn_port can be used.

Prototype:

sint8 espconn_secure_connect (struct espconn *espconn)

Parameters:

struct espconn *espconn : corresponding connected control block structure

Return:

0	: succeed

Non-0 : error code ESPCONN_ARG - illegal argument，can’t find TCP connection according to structure espconn

### espconn_secure_sent

Function: send encrypted data (SSL)

Note:

Please call espconn_secure_sent after espconn_sent_callback of the pre-packet.

Prototype:

sint8 espconn_secure_sent ( struct espconn *espconn, uint8 *psent,

uint16 length

)

Parameters:

struct espconn *espconn : corresponding connected control block structure uint8 *psent : sent data pointer

uint16 length : sent data length

Return:

0	: succeed

Non-0 : error code ESPCONN_ARG - illegal argument，can’t find TCP connection according to structure espconn

### espconn_secure_disconnect

Function: secure TCP disconnection(SSL)

Prototype:

sint8 espconn_secure_disconnect(struct espconn *espconn)

Parameters:

struct espconn *espconn : corresponding connected control block structure

Return:

0	: succeed

Non-0 : error code ESPCONN_ARG - illegal argument，can’t find TCP connection according to structure espconn

### espconn_tcp_get_max_con

Function:

Get maximum number of how many TCP connection is allowed.

Prototype:

uint8 espconn_tcp_get_max_con(void)

Parameter: null

Return:

Maximum number of how many TCP connection is allowed.

### espconn_tcp_set_max_con

Function:

Set the maximum number of how many TCP connection is allowed.

Prototype:

sint8 espconn_tcp_set_max_con(uint8 num)

Parameter:

uint8 num : Maximum number of how many TCP connection is allowed.

Return:

0	: succeed

Non-0 : error code ESPCONN_ARG - illegal argument，can’t find TCP connection according to structure espconn

### espconn_tcp_get_max_con_allow

Function:

Get the maximum number of TCP clients which are allowed to connect to ESP8266 TCP server.

Prototype:

sint8 espconn_tcp_get_max_con_allow(struct espconn *espconn)

Parameter:

struct espconn *espconn : corresponding connected control structure

Return:

> 0 : Maximum number of TCP clients which are allowed.

< 0 : error code ESPCONN_ARG - illegal argument，can’t find TCP connection according to structure espconn

### espconn_tcp_set_max_con_allow

Function:

Set the maximum number of TCP clients allowed to connect to ESP8266 TCP server.

Prototype:

sint8 espconn_tcp_set_max_con_allow(struct espconn *espconn, uint8 num)

Parameter:

struct espconn *espconn : corresponding connected control structure uint8 num : Maximum number of TCP clients which are allowed.

Return:

0	: succeed

Non-0 : error code ESPCONN_ARG - illegal argument，can’t find TCP connection according to structure espconn

### espconn_recv_hold

Function:

Puts in a request to block the TCP receive function.

Note:

The function does not act immediately; we recommend calling it while reserving 5*1460 bytes of memory.

This API can be called more than once.

Prototype:

sint8 espconn_recv_hold(struct espconn *espconn)

Parameter:

struct espconn *espconn : corresponding connected control structure

Return:

0	: succeed

Non-0 : error code ESPCONN_ARG - illegal argument，can’t find TCP connection according to structure espconn

### espconn_recv_unhold

Function:

Unblock TCP receiving data (i.e. undo espconn_recv_hold).

Note:

This API takes effect immediately.

Prototype:

sint8 espconn_recv_unhold(struct espconn *espconn)

Parameter:

struct espconn *espconn : corresponding connected control structure

Return:

0	: succeed

Non-0 : error code ESPCONN_ARG - illegal argument，can’t find TCP connection according to structure espconn

## UDP APIs

### espconn_create

Function: create UDP transmission.

Prototype:

sin8 espconn_create(struct espconn *espconn)

Parameter:

struct espconn *espconn : corresponding connected control block structure

Return:

0	: succeed Non-0 : error code

ESPCONN_ISCONN - Already connected ESPCONN_MEM - Out of memory

ESPCONN_ARG - illegal argument，can’t find UDP transmission according to

structure espconn

### espconn_igmp_join

Function:

Join a multicast group

Prototype:

sint8 espconn_igmp_join(ip_addr_t *host_ip, ip_addr_t *multicast_ip)

Parameters:

ip_addr_t *host_ip : IP of host

ip_addr_t *multicast_ip : IP of multicast group

Return:

0	: succeed

Non-0 : error code ESPCONN_MEM - Out of memory

### espconn_igmp_leave

Function:

Quit a multicast group

Prototype:

sint8 espconn_igmp_leave(ip_addr_t *host_ip, ip_addr_t *multicast_ip)

Parameters:

ip_addr_t *host_ip

: IP of host

ip_addr_t *multicast_ip : IP of multicast group

Return:

0	: succeed

Non-0 : error code ESPCONN_MEM - Out of memory

## mDNS APIs

### espconn_mdns_init

Function:

mDNS initialization

Note：

- Only ESP8266 station support mDNS, please get IP address of ESP8266 station first，then call this API to initial mDNS；

- txt_data has to be set as “ key = value ”，as Example；

Structure:

struct mdns_info{ char *host_name; char *server_name;

uint16 server_port; unsigned long ipAddr;

char *txt_data;

};

Prototype:

void espconn_mdns_init(struct mdns_info *info)

Parameter:

struct mdns_info *info : mdns information

Return:

none

Example：

struct mdns_info *info = (struct mdns_info *)os_zalloc(sizeof(struct mdns_info));

info->host_name = “espressif”;

info->ipAddr = station_ipconfig.ip.addr; //ESP8266 station IP info->server_name = “iot”;

info->server_port = 8080;

info->txt_data = “version = now”; espconn_mdns_init(info);

### espconn_mdns_close

Function:

close mDNS, corresponding creation API : espconn_mdns_init

Prototype:

void espconn_mdns_close(void)

Parameter: none

Return:

none

### espconn_mdns_server_register

Function:

register mDNS server

Prototype:

void espconn_mdns_server_register(void)

Parameter: none

Return:

none

### espconn_mdns_server_unregister

Function:

unregister mDNS server

Prototype:

void espconn_mdns_server_unregister(void)

Parameter: none

Return:

none

### espconn_mdns_get_servername

Function:

Get mDNS server name

Prototype:

char* espconn_mdns_get_servername(void)

Parameter: none

Return:

server name

### espconn_mdns_set_servername

Function:

Set mDNS server name

Prototype:

void espconn_mdns_set_servername(const char *name)

Parameter:

const char *name - server name

Return:

none

### espconn_mdns_set_hostname

Function:

Set mDNS host name

Prototype:

void espconn_mdns_set_hostname(char *name)

Parameter:

char *name - host name

Return:

none

### espconn_mdns_get_hostname

Function:

Get mDNS host name

Prototype:

char* espconn_mdns_get_hostname(void)

Parameter: none

Return:

host name

### espconn_mdns_disable

Function:

Disable mDNS , corresponding creation API : espconn_mdns_enable

Prototype:

void espconn_mdns_disable(void)

Parameter: none

Return:

none

### espconn_mdns_enable

Function:

Enable mDNS

Prototype:

void espconn_mdns_enable(void)

Parameter: none

Return:

none

# Application Related

## AT APIs

for AT APIs examples, refer to esp_iot_sdk/examples/at/user/user_main.c.

### at_response_ok

Function:

Output OK to AT Port (UART0)

Prototype:

void at_response_ok(void)

Parameter: null

Return:

null

### at_response_error

Function:

output ERROR to AT Port (UART0)

Prototype:

void at_response_error(void)

Parameter: null

Return:

null

### at_cmd_array_regist

Function:

register user-define AT commands.

Can be called only once to register all user-define AT commands.

Prototype:

void at_cmd_array_regist (

at_function * custom_at_cmd_arrar, uint32 cmd_num

)

Parameter:

at_function * custom_at_cmd_arrar : Array of user-define AT commands uint32 cmd_num : Number counts of user-define AT commands

Return:

null

Example:

refer to esp_iot_sdk/examples/at/user/user_main.c

### at_get_next_int_dec

Function:

parse int from AT command

Prototype:

bool at_get_next_int_dec (char **p_src,int* result,int* err)

Parameter:

char **p_src : *p_src is the AT command that need to be parsed int* result : int number parsed from the AT command

int* err	: 1: no number is found; 3: only ’-’ is found.

Return:

true: parser succeeds (NOTE: if no number is found, it will return True, but returns error code 1)

false: parser is unable to parse string; some probable causes are: int number more than 10 bytes; string contains termination characters '/r'; string contains only '-'.

Example:

refer to esp_iot_sdk/examples/at/user/user_main.c

### at_data_str_copy

Function: parse string from AT command

Prototype:

int32 at_data_str_copy (char * p_dest, char ** p_src,int32 max_len)

Parameter:

char * p_dest : string parsed from the AT command

char ** p_src : *p_src is the AT command that need to be parsed int32 max_len : max string length that allowed

Return:

length of string:

>=0: succeed and returns the length of the string

<0 : fail and returns -1

Example:

refer to esp_iot_sdk/examples/at/user/user_main.c

### at_init

Function:

AT initialize

Prototype:

void at_init (void)

Parameter: null

Return:

null

Example:

refer to esp_iot_sdk/examples/at/user/user_main.c

### at_port_print

Function:

output string to AT PORT(UART0)

Prototype:

void at_port_print(const char *str)

Parameter:

const char *str : string that need to output

Return:

null

Example:

refer to esp_iot_sdk/examples/at/user/user_main.c

### at_set_custom_info

Function:

User-define version info of AT which can be got by AT+GMR.

Prototype:

void at_set_custom_info (char *info)

Parameter:

char *info : version info

Return:

null

### at_enter_special_state

Function:

Enter processing state. In processing state, AT core will return busy for any further AT commands.

Prototype:

void at_enter_special_state (void)

Parameter: null

Return:

null

### at_leave_special_state

Function:

Exit from AT processing state.

Prototype:

void at_leave_special_state (void)

Parameter: null

Return:

null

### at_get_version

Function:

Get Espressif AT lib version.

Prototype:

uint32 at_get_version (void)

Parameter: null

Return:

Espressif AT lib version

### at_register_uart_rx_intr

Function:

Set UART0 to be used by user or AT commands.

Note:

This API can be called multiple times.

Running AT，UART0 default to be used by AT commands.

Prototype:

void at_register_uart_rx_intr(at_custom_uart_rx_intr rx_func)

Parameter:

at_custom_uart_rx_intr : register a UART0 RX interrupt handler so that UART0 can be used by the customer, while if it’s NULL, UART0 is assigned to AT commands.

Return:

null

Example:

void user_uart_rx_intr(uint8* data, int32 len)

{

// UART0 rx for user os_printf(“len=%d \r\n”,len); os_printf(data);

// change UART0 for AT at_register_uart_rx_intr(NULL);

}

void user_init(void){ at_register_uart_rx_intr(user_uart_rx_intr); }

## Related JSON APIs

Found in : esp_iot_sdk/include/json/jsonparse.h & jsontree.h

### jsonparse_setup

Function:

json initialize parsing

Prototype:

void jsonparse_setup(

struct jsonparse_state *state, const char *json,

int len

)

Parameters:

struct jsonparse_state *state : json parsing pointer const char *json : json parsing character string int len : character string length

Return:

null

### jsonparse_next

Function:

Returns jsonparse next object

Prototype:

int jsonparse_next(struct jsonparse_state *state)

Parameters:

struct jsonparse_state *state : json parsing pointer

Return:

int : parsing result

### jsonparse_copy_value

Function:

Copies current parsing character string to a certain buffer

Prototype:

int jsonparse_copy_value(

struct jsonparse_state *state, char *str,

int size

)

Parameters:

struct jsonparse_state *state : json parsing pointer char *str : buffer pointer

int size : buffer size

Return:

int : copy result

### jsonparse_get_value_as_int

Function:

Parses json to get integer

Prototype:

int jsonparse_get_value_as_int(struct jsonparse_state *state)

Parameters:

struct jsonparse_state *state : json parsing pointer

Return:

int : parsing result

### jsonparse_get_value_as_long

Function:

Parses json to get long integer

Prototype:

long jsonparse_get_value_as_long(struct jsonparse_state *state)

Parameters:

struct jsonparse_state *state : json parsing pointer

Return:

long : parsing result

### jsonparse_get_len

Function:

Gets parsed json length

Prototype:

int jsonparse_get_value_len(struct jsonparse_state *state)

Parameters:

struct jsonparse_state *state : json parsing pointer

Return:

int : parsed jason length

### jsonparse_get_value_as_type

Function:

Parses json data type

Prototype:

int jsonparse_get_value_as_type(struct jsonparse_state *state)

Parameters:

struct jsonparse_state *state : json parsing pointer

Return:

int : parsed json data type

### jsonparse_strcmp_value

Function:

Compares parsed json and certain character string

Prototype:

int jsonparse_strcmp_value(struct jsonparse_state *state, const char *str)

Parameters:

struct jsonparse_state *state : json parsing pointer const char *str : character buffer

Return:

int : comparison result

### jsontree_set_up

Function:

Creates json data tree

Prototype:

void jsontree_setup(

struct jsontree_context *js_ctx, struct jsontree_value *root,

int (* putchar)(int)

)

Parameters:

struct jsontree_context *js_ctx : json tree element pointer struct jsontree_value *root : root element pointer

int (* putchar)(int) : input function

Return:

null

### jsontree_reset

Function:

Resets json tree

Prototype:

void jsontree_reset(struct jsontree_context *js_ctx)

Parameters:

struct jsontree_context *js_ctx : json data tree pointer

Return:

null

### jsontree_path_name

Function:

get json tree parameters

Prototype:

const char *jsontree_path_name(

const struct jsontree_cotext *js_ctx, int depth

)

Parameters:

struct jsontree_context *js_ctx : json tree pointer int depth : json tree depth

Return:

char* : parameter pointer

### jsontree_write_int

Function:

write integer to json tree

Prototype:

void jsontree_write_int(

const struct jsontree_context *js_ctx, int value

)

Parameters:

struct jsontree_context *js_ctx : json tree pointer int value : integer value

Return:

null

### jsontree_write_int_array

Function:

Writes integer array to json tree

Prototype:

void jsontree_write_int_array(

const struct jsontree_context *js_ctx, const int *text,

uint32 length

)

Parameters:

struct jsontree_context *js_ctx : json tree pointer int *text : array entry address

uint32 length : array length

Return:

null

### jsontree_write_string

Function:

Writes string to json tree

Prototype:

void jsontree_write_string(

const struct jsontree_context *js_ctx, const char *text

)

Parameters:

struct jsontree_context *js_ctx : json tree pointer const char* text : character string pointer

Return:

null

### jsontree_print_next

Function:

json tree depth

Prototype:

int jsontree_print_next(struct jsontree_context *js_ctx)

Parameters:

struct jsontree_context *js_ctx : json tree pointer

Return:

int : json tree depth

### jsontree_find_next

Function:

find json tree element

Prototype:

struct jsontree_value *jsontree_find_next( struct jsontree_context *js_ctx, int type

)

Parameters:

struct jsontree_context *js_ctx : json tree pointer int : type

Return:

struct jsontree_value * : json tree element pointer

# Definition of Structures

## Timer

typedef void ETSTimerFunc(void *timer_arg); typedef struct _ETSTIMER_ {

struct _ETSTIMER_	*timer_next; uint32_t		timer_expire;

uint32_t	timer_period;

ETSTimerFunc	*timer_func;

void	*timer_arg;

} ETSTimer;

## WiFi Related Structures

### Station Related

struct station_config { uint8 ssid[32]; uint8 password[64]; uint8 bssid_set; uint8 bssid[6];

};

Note:

BSSID as MAC address of AP, will be used when several APs have the same SSID.

If station_config.bssid_set==1 , station_config.bssid has to be set, otherwise, the connection will fail.

In general, station_config.bssid_set need to be 0.

### soft-AP related

typedef enum _auth_mode { AUTH_OPEN = 0, AUTH_WEP, AUTH_WPA_PSK, AUTH_WPA2_PSK, AUTH_WPA_WPA2_PSK

} AUTH_MODE;

struct softap_config {

uint8 ssid[32]; uint8 password[64]; uint8 ssid_len;

uint8 channel;	// support 1 ~ 13

uint8 authmode;	// Don’t support AUTH_WEP in soft-AP mode uint8 ssid_hidden;	// default 0

uint8 max_connection;	// default 4, max 4

uint16 beacon_interval; // 100 ~ 60000 ms, default 100

};

Note:

If softap_config.ssid_len==0, check ssid till find a termination characters; otherwise, it depends on softap_config.ssid_len.

### scan related

struct scan_config { uint8 *ssid; uint8 *bssid; uint8 channel;

uint8 show_hidden; // Scan APs which are hiding their SSID or not.

};

struct bss_info { STAILQ_ENTRY(bss_info) next; u8 bssid[6];

u8 ssid[32]; u8 channel; s8 rssi;

u8 authmode;

uint8 is_hidden; // SSID of current AP is hidden or not.

};

typedef void (* scan_done_cb_t)(void *arg, STATUS status);

### WiFi event related structure

enum {

EVENT_STAMODE_CONNECTED = 0, EVENT_STAMODE_DISCONNECTED, EVENT_STAMODE_AUTHMODE_CHANGE, EVENT_STAMODE_GOT_IP, EVENT_SOFTAPMODE_STACONNECTED,

EVENT_SOFTAPMODE_STADISCONNECTED,

| }; | EVENT_MAX |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| enum | { REASON_UNSPECIFIED | = | 1, |  |  |  |
|  | REASON_AUTH_EXPIRE | = | 2, |  |  |  |
|  | REASON_AUTH_LEAVE | = | 3, |  |  |  |
|  | REASON_ASSOC_EXPIRE | = | 4, |  |  |  |
|  | REASON_ASSOC_TOOMANY | = | 5, |  |  |  |
|  | REASON_NOT_AUTHED | = | 6, |  |  |  |
|  | REASON_NOT_ASSOCED | = | 7, |  |  |  |
|  | REASON_ASSOC_LEAVE | = | 8, |  |  |  |
|  | REASON_ASSOC_NOT_AUTHED | = | 9, |  |  |  |
|  | REASON_DISASSOC_PWRCAP_BAD | = | 10, | /* | 11h | */ |
|  | REASON_DISASSOC_SUPCHAN_BAD | = | 11, | /* | 11h | */ |
|  | REASON_IE_INVALID | = | 13, | /* | 11i | */ |
|  | REASON_MIC_FAILURE | = | 14, | /* | 11i | */ |
|  | REASON_4WAY_HANDSHAKE_TIMEOUT | = | 15, | /* | 11i | */ |
|  | REASON_GROUP_KEY_UPDATE_TIMEOUT | = | 16, | /* | 11i | */ |
|  | REASON_IE_IN_4WAY_DIFFERS | = | 17, | /* | 11i | */ |
|  | REASON_GROUP_CIPHER_INVALID | = | 18, | /* | 11i | */ |
|  | REASON_PAIRWISE_CIPHER_INVALID | = | 19, | /* | 11i | */ |
|  | REASON_AKMP_INVALID | = | 20, | /* | 11i | */ |
|  | REASON_UNSUPP_RSN_IE_VERSION | = | 21, | /* | 11i | */ |
|  | REASON_INVALID_RSN_IE_CAP | = | 22, | /* | 11i | */ |
|  | REASON_802_1X_AUTH_FAILED | = | 23, | /* | 11i | */ |
|  | REASON_CIPHER_SUITE_REJECTED | = | 24, | /* | 11i | */ |
|  | REASON_BEACON_TIMEOUT | = | 200, |  |  |  |
| }; | REASON_NO_AP_FOUND | = | 201, |  |  |  |

typedef struct {

uint8 ssid[32]; uint8 ssid_len; uint8 bssid[6]; uint8 channel;

} Event_StaMode_Connected_t;

typedef struct {

uint8 ssid[32]; uint8 ssid_len; uint8 bssid[6]; uint8 reason;

} Event_StaMode_Disconnected_t;

typedef struct {

uint8 old_mode; uint8 new_mode;

} Event_StaMode_AuthMode_Change_t;

typedef struct {

struct ip_addr ip; struct ip_addr mask; struct ip_addr gw;

} Event_StaMode_Got_IP_t;

typedef struct {

uint8 mac[6]; uint8 aid;

} Event_SoftAPMode_StaConnected_t;

typedef struct {

uint8 mac[6]; uint8 aid;

} Event_SoftAPMode_StaDisconnected_t;

typedef union {

Event_StaMode_Connected_t	connected;

Event_StaMode_Disconnected_t	disconnected; Event_StaMode_AuthMode_Change_t	auth_change; Event_StaMode_Got_IP_t		got_ip; Event_SoftAPMode_StaConnected_t	sta_connected; Event_SoftAPMode_StaDisconnected_t	sta_disconnected;

} Event_Info_u;

typedef struct _esp_event {

uint32 event; Event_Info_u event_info;

} System_Event_t;

typedef enum {

SC_STATUS_WAIT = 0,	// Please don’t start connection in this phase SC_STATUS_FIND_CHANNEL,	// Start connection by APP in this phase SC_STATUS_GETTING_SSID_PSWD,

SC_STATUS_GOT_SSID_PSWD, SC_STATUS_LINK,

SC_STATUS_LINK_OVER,	// Got IP, connect to AP successfully

} sc_status; typedef enum {

SC_TYPE_ESPTOUCH = 0, SC_TYPE_AIRKISS,

} sc_type;

smart config structure

## JSON Related Structure

### json structure

struct jsontree_value { uint8_t type;

};

struct jsontree_pair { const char *name;

struct jsontree_value *value;

};

struct jsontree_context {

struct jsontree_value *values[JSONTREE_MAX_DEPTH]; uint16_t index[JSONTREE_MAX_DEPTH];

int (* putchar)(int); uint8_t depth; uint8_t path;

int callback_state;

};

struct jsontree_callback { uint8_t type;

int (* output)(struct jsontree_context *js_ctx); int (* set)(struct jsontree_context *js_ctx,

struct jsonparse_state *parser);

};

struct jsontree_object { uint8_t type; uint8_t count;

struct jsontree_pair *pairs;

};

struct jsontree_array { uint8_t type; uint8_t count;

struct jsontree_value **values;

};

struct jsonparse_state { const char *json; int pos;

int len; int depth; int vstart; int vlen; char vtype; char error;

char stack[JSONPARSE_MAX_DEPTH];

};

### json macro definition

#define JSONTREE_OBJECT(name, ...)

static struct jsontree_pair jsontree_pair_##name[] = { VA_ARGS }; static struct jsontree_object name = {

JSON_TYPE_OBJECT,

sizeof(jsontree_pair_##name)/sizeof(struct jsontree_pair),

/

/

/

/

/

jsontree_pair_##name }

#define JSONTREE_PAIR_ARRAY(value) (struct jsontree_value *)(value) #define JSONTREE_ARRAY(name, ...)	/

static struct jsontree_value* jsontree_value_##name[] = { VA_ARGS }; / static struct jsontree_array name = {		/ JSON_TYPE_ARRAY,	/

sizeof(jsontree_value_##name)/sizeof(struct jsontree_value*),	/

jsontree_value_##name }

## espconn parameters

### callback function

/** callback prototype to inform about events for a espconn */

typedef void (* espconn_recv_callback)(void *arg, char *pdata, unsigned short len);

typedef void (* espconn_callback)(void *arg, char *pdata, unsigned short len);

typedef void (* espconn_connect_callback)(void *arg);

### espconn

typedef void* espconn_handle; typedef struct _esp_tcp {

int remote_port; int local_port; uint8 local_ip[4]; uint8 remote_ip[4];

espconn_connect_callback connect_callback; espconn_reconnect_callback reconnect_callback; espconn_connect_callback disconnect_callback; espconn_connect_callback write_finish_fn;

} esp_tcp;

typedef struct _esp_udp { int remote_port;

int local_port; uint8 local_ip[4]; uint8 remote_ip[4];

} esp_udp;

/** Protocol family and type of the espconn */ enum espconn_type {

ESPCONN_INVALID	= 0,

/* ESPCONN_TCP Group */ ESPCONN_TCP	= 0x10,

/* ESPCONN_UDP Group */ ESPCONN_UDP	= 0x20,

};

/** Current state of the espconn. Non-TCP espconn are always in state ESPCONN_NONE! */

enum espconn_state { ESPCONN_NONE, ESPCONN_WAIT, ESPCONN_LISTEN, ESPCONN_CONNECT, ESPCONN_WRITE, ESPCONN_READ, ESPCONN_CLOSE

};

enum espconn_option{

ESPCONN_START = 0x00， ESPCONN_REUSEADDR = 0x01， ESPCONN_NODELAY = 0x02， ESPCONN_COPY = 0x04， ESPCONN_KEEPALIVE = 0x08， ESPCONN_END

}

enum espconn_level{

ESPCONN_KEEPIDLE, ESPCONN_KEEPINTVL, ESPCONN_KEEPCNT

}

/** A espconn descriptor */ struct espconn {

/** type of the espconn (TCP, UDP) */ enum espconn_type type;

/** current state of the espconn */ enum espconn_state state;

union {

esp_tcp *tcp; esp_udp *udp;

} proto;

/** A callback function that is informed about events for this espconn */ espconn_recv_callback recv_callback;

espconn_sent_callback sent_callback; uint8 link_cnt;

void *reverse; // reversed for customer use

};

# Peripheral Related Drivers

## GPIO Related APIs

Please refer to /user/user_plug.c.

### PIN Related Macros

The following macros are used to control the GPIO pins' status.

PIN_PULLUP_DIS(PIN_NAME)

Disable pin pull up

PIN_PULLUP_EN(PIN_NAME)

Enable pin pull up

PIN_FUNC_SELECT(PIN_NAME, FUNC)

Select pin function

Example:

PIN_FUNC_SELECT(PERIPHS_IO_MUX_MTDI_U, FUNC_GPIO12); // Use MTDI pin as GPIO12.

### gpio_output_set

Function: set gpio property

Prototype:

void gpio_output_set( uint32 set_mask, uint32 clear_mask, uint32 enable_mask, uint32 disable_mask

)

Input Parameters:

uint32 set_mask : set high output; 1:high output; 0:no status change uint32 clear_mask : set low output; 1:low output; 0:no status change uint32 enable_mask : enable outpout bit

uint32 disable_mask : enable input bit

Return:

null

Example:

gpio_output_set(BIT12, 0, BIT12, 0): Set GPIO12 as high-level output; gpio_output_set(0, BIT12, BIT12, 0):

Set GPIO12 as low-level output gpio_output_set(BIT12, BIT13, BIT12|BIT13, 0):

Set GPIO12 as high-level output, GPIO13 as low-level output. gpio_output_set(0, 0, 0, BIT12):

Set GPIO12 as input

### GPIO input and output macro

GPIO_OUTPUT_SET(gpio_no, bit_value)

Set gpio_no as output bit_value, the same as the output example in 5.1.2

GPIO_DIS_OUTPUT(gpio_no)

Set gpio_no as input, the same as the input example in 5.1.2.

GPIO_INPUT_GET(gpio_no)

Get the level status of gpio_no.

### GPIO interrupt

ETS_GPIO_INTR_ATTACH(func, arg)

Register GPIO interrupt control function

ETS_GPIO_INTR_DISABLE()

Disable GPIO interrupt

ETS_GPIO_INTR_ENABLE()

Enable GPIO interrupt

### gpio_pin_intr_state_set

Function:

set GPIO interrupt state

Prototype:

void gpio_pin_intr_state_set( uint32 i,

GPIO_INT_TYPE intr_state

)

| GPIO_PIN_INTR_DISABLE | = | 0, |
| --- | --- | --- |
| GPIO_PIN_INTR_POSEDGE | = | 1, |
| GPIO_PIN_INTR_NEGEDGE | = | 2, |
| GPIO_PIN_INTR_ANYEGDE | = | 3, |
| GPIO_PIN_INTR_LOLEVEL | = | 4, |
| GPIO_PIN_INTR_HILEVEL | = | 5 |

Input Parameters:

uint32 i : GPIO pin ID, if you want to set GPIO14, pls use GPIO_ID_PIN(14); GPIO_INT_TYPE intr_state : interrupt type as the following:

typedef enum {

} GPIO_INT_TYPE;

Return:

null

GPIO Interrupt Handler

Follow the steps below to clear interrupt status in GPIO interrupt processing function:

uint32 gpio_status;

gpio_status = GPIO_REG_READ(GPIO_STATUS_ADDRESS);

//clear interrupt status GPIO_REG_WRITE(GPIO_STATUS_W1TC_ADDRESS, gpio_status);

## UART Related APIs

By default, UART0 is debug output interface. In the case of dual Uart, UART0 works as data receive and transmit interface, and UART1as debug output interface.

Please make sure all hardware are correctly connected.

### uart_init

Function:

Initializes baud rates of the two uarts

Prototype:

void uart_init( UartBautRate uart0_br, UartBautRate uart1_br

)

Parameters:

UartBautRate uart0_br : uart0 baud rate UartBautRate uart1_br : uart1 baud rate

| BIT_RATE_9600 | = | 9600, |
| --- | --- | --- |
| BIT_RATE_19200 | = | 19200, |
| BIT_RATE_38400 | = | 38400, |
| BIT_RATE_57600 | = | 57600, |
| BIT_RATE_74880 | = | 74880, |
| BIT_RATE_115200 | = | 115200, |
| BIT_RATE_230400 | = | 230400, |
| BIT_RATE_460800 | = | 460800, |
| BIT_RATE_921600 | = | 921600 |

Baud Rates:

typedef enum {

} UartBautRate;

Return:

null

uart0_tx_buffer

Function:

Sends user-defined data through UART0

Prototype:

void uart0_tx_buffer(uint8 *buf, uint16 len)

Parameter:

uint8 *buf : data to send later

uint16 len : the length of data to send later

Return:

null

### uart0_rx_intr_handler

Function:

UART0 interrupt processing function. Users can add the processing of received data in this function. (Receive buffer size: 0x100; if the received data are more than 0x100, pls handle them yourselves.)

Prototype:

void uart0_rx_intr_handler(void *para)

Parameter:

void *para : the pointer pointing to RcvMsgBuff structure

Return:

null

## I2C Master Related APIs

### i2c_master_gpio_init

Function:

Set GPIO in I2C master mode

Prototype:

void i2c_master_gpio_init (void)

Input Parameters: null

Return:

null

### i2c_master_init

Function:

Initialize I2C

Prototype:

void i2c_master_init(void)

Input Parameters: null

Return:

null

### i2c_master_start

Function: configures I2C to start sending data

Prototype:

void i2c_master_start(void)

Input Parameters: null

Return:

null

### i2c_master_stop

Function:

configures I2C to stop sending data

Prototype:

void i2c_master_stop(void)

Input Parameters: null

Return:

null

### i2c_master_send_ack

Function:

Sends I2C ACK

Prototype:

void i2c_master_send_ack (void)

Input Parameters: null

Return:

null

### i2c_master_send_nack

Function:

Sends I2C NACK

Prototype:

void i2c_master_send_nack (void)

Input Parameters: null

Return:

null

### i2c_master_checkAck

Function:

Checks ACK from slave

Prototype:

bool i2c_master_checkAck (void)

Input Parameters: null

Return:

true: get I2C slave ACK false: get I2C slave NACK

### i2c_master_readByte

Function:

Read one byte from I2C slave

Prototype:

uint8 i2c_master_readByte (void)

Input Parameters: null

Return:

uint8 : the value that was read

### i2c_master_writeByte

Function:

Write one byte to slave

Prototype:

void i2c_master_writeByte (uint8 wrdata)

Input Parameters:

uint8 wrdata : data to write

Return:

null

## PWM Related

ESP9266 supports 4x PWM outputs. More details can be found in pwm.h. It is possible to increase the number of PWM outputs, but it is beyond the scope of this document.

### pwm_init

Function:

Initialize PWM function, including GPIO selection, frequency and duty cycle.

Prototype:

void pwm_init(uint16 freq, uint8 *duty)

Input Parameters:

uint16 freq : PWM frequency;

uint8 *duty : duty cycle of each output

Return:

null

### pwm_start

Function:

Starts PWM. This function needs to be called after PWM config is changed.

Prototype:

void pwm_start (void)

Parameter: null

Return:

null

### pwm_set_duty

Function:

Sets duty cycle of an output

Prototype:

void pwm_set_duty(uint8 duty, uint8 channel)

Input Parameters:

uint8 duty : duty cycle uint8 channel : an output

Return:

null

### pwm_set_freq

Function:

Sets PWM frequency

Prototype:

void pwm_set_freq(uint16 freq)

Input Parameters:

uint16 freq : PWM frequency

Return:

null

### pwm_get_duty

Function:

Gets duty cycle of PWM output

Prototype:

uint8 pwm_get_duty(uint8 channel)

Input Parameters:

uint8 channel : channel of which to get duty cycle

Return:

uint8 : duty cycle

### pwm_get_freq

Function:

Gets PWM frequency.

Prototype:

uint16 pwm_get_freq(void)

Input Parameters: null

Return:

uint16 : frequency

# Appendix

## ESPCONN Programming

### TCP Client Mode

Notes

- ESP8266, работающий в режиме станции, начнет клиентское соединение при получении IP-адреса.

- ESP8266, работающая в режиме soft-AP, начнет клиентское подключение, когда устройствам, подключенным к ESP8266, будет присвоен IP-адрес.

Steps

- Initialize espconn parameters according to protocols.

- Зарегистрируйте функцию обратного вызова connect и зарегистрируйте функцию обратного вызова reconnect.

  - (Call espconn_regist_connectcb and espconn_regist_reconcb )

- Call espconn_connect function and set up the connection with TCP Server.

- Зарегистрированная подключенная функция обратного вызова будет вызвана после успешного подключения, которая зарегистрирует соответствующую функцию обратного вызова. Рекомендую зарегистрировать функцию обратного вызова отключения.

  - (Call espconn_regist_recvcb , espconn_regist_sentcb and espconn_regist_disconcb

in connected callback)

- При использовании функции приема обратного вызова или функции обратного вызова для запуска отключения рекомендуется установить временную задержку, чтобы убедиться, что все функции прошивки завершены.

### TCP Server Mode

Notes

- Если ESP8266 находится в режиме станции, он начнет прослушивание сервера при получении IP-адреса.

- Если ESP8266 находится в режиме soft-AP, он начнет прослушивание сервера.

Steps

- Initialize espconn parameters according to protocols.

- Register connect callback and reconnect callback function.

  - (Call espconn_regist_connectcb and espconn_regist_reconcb )

- Call espconn_accept function to listen to the connection with host.

- Registered connect function will be called after successful connection, which will register corresponding callback function.

  - (Call espconn_regist_recvcb , espconn_regist_sentcb and espconn_regist_disconcb

in connected callback)

## RTC APIs Example

Demo code below shows how to get RTC time and to read and write to RTC memory.

void user_init(void) {

os_printf(clk cal : %d /n/r,system_rtc_clock_cali_proc()>>12);

uint32 rtc_time = 0, rtc_reg_val = 0,stime = 0,rtc_time2 = 0,stime2 = 0; rtc_time = system_get_rtc_time();

stime = system_get_time();

os_printf(rtc time : %d /n/r,rtc_time); os_printf(system time : %d /n/r,stime);

if( system_rtc_mem_read(0, &rtc_reg_val, 4)) { os_printf(rtc mem val : 0x%08x/n/r,rtc_reg_val);

} else {

os_printf(rtc mem val error/n/r);

}

rtc_reg_val++;

os_printf(rtc mem val write/n/r); system_rtc_mem_write(0, &rtc_reg_val, 4) ;

if( system_rtc_mem_read(0, &rtc_reg_val, 4) ){ os_printf(rtc mem val : 0x%08x/n/r,rtc_reg_val);

} else {

os_printf(rtc mem val error/n/r);

}

rtc_time2 = system_get_rtc_time(); stime2 = system_get_time();

os_printf(rtc time : %d /n/r,rtc_time2); os_printf(system time : %d /n/r,stime2); os_printf(delta time rtc: %d /n/r,rtc_time2-rtc_time);

os_printf(delta system time rtc: %d /n/r,stime2-stime); os_printf(clk cal : %d /n/r,system_rtc_clock_cali_proc()>>12);

os_delay_us(500000); system_restart();

}

## Sniffer Structure Introduction

ESP8266 can enter promiscuous mode (sniffer) and capture IEEE 802.11 packets in the air. The following HT20 packets are support:

- 802.11b

- 802.11g

- 802.11n (from MCS0 to MCS7)

- AMPDU types of packets The following are not supported:

- HT40

- LDPC

Although ESP8266 can not completely decipher these kinds of IEEE80211 packets completely, it can still obtain the length of these special packets.

In summary, while in sniffer mode, ESP8266 can either capture completely the packets or obtain the length of the packet:

- Packets that ESP8266 can decipher completely; ESP8266 returns with the

  - MAC address of the both side of communication and encryption type and

  - the length of entire packet.

- Packets that ESP8266 can only partial decipher; ESP8266 returns with

  - the length of packet.

Structure RxControl and sniffer_buf are used to represent these two kinds of packets. Structure

sniffer_buf contains structure RxControl.

struct RxControl { signed rssi:8; unsigned rate:4; unsigned is_group:1;

unsigned:1;

// signal intensity of packet

unsigned sig_mode:2;	// 0:is 11n packet; 1:is not 11n packet; unsigned legacy_length:12; // if not 11n packet, shows length of packet. unsigned damatch0:1;

unsigned damatch1:1; unsigned bssidmatch0:1; unsigned bssidmatch1:1;

unsigned MCS:7;	// if is 11n packet, shows the modulation

// and code used (range from 0 to 76) unsigned CWB:1; // if is 11n packet, shows if is HT40 packet or not unsigned HT_length:16;// if is 11n packet, shows length of packet. unsigned Smoothing:1;

unsigned Not_Sounding:1; unsigned:1;

unsigned Aggregation:1; unsigned STBC:2;

unsigned FEC_CODING:1; // if is 11n packet, shows if is LDPC packet or not. unsigned SGI:1;

unsigned rxend_state:8; unsigned ampdu_cnt:8;

unsigned channel:4; //which channel this packet in. unsigned:12;

};

struct LenSeq{

u16 len; // length of packet

u16 seq; // serial number of packet, the high 12bits are serial number,

//	low 14 bits are Fragment number (usually be 0) u8 addr3[6]; // the third address in packet

};

struct sniffer_buf{

struct RxControl rx_ctrl;

u8 buf[36 ]; // head of ieee80211 packet u16 cnt;	// number count of packet struct LenSeq lenseq[1]; //length of packet

};

Callback wifi_promiscuous_rx has two parameters ( buf and len). len means the length of buf, len = 12 or len ≥ 60:

Case of LEN ≥ 60

- buf contains structure sniffer_buf: this structure is reliable, data packets represented by it has been verified by CRC.

- sniffer_buf.cnt means the count of packets in buf. The value of len depends on

sniffer_buf.cnt.

  - sniffer_buf.cnt==0, invalid buf; otherwise, len = 50 + cnt * 10

- sniffer_buf.buf contains the first 36 bytes of ieee80211 packet. Starting from sniffer_buf.lenseq[0], each structure lenseq represent a length information of packet. lenseq[0] represents the length of first packet. If there are two packets where (sniffer_buf.cnt == 2), lenseq[1] represents the length of second packet.

- If sniffer_buf.cnt > 1, it is a AMPDU packet, head of each MPDU packets are similar, so we only provide the length of each packet (from head of MAC packet to FCS)

- This structure contains: length of packet, MAC address of both sides of communication, length of the head of packet.

Case of LEN==12

- buf contains structure RxControl; but this structure is not reliable, we can not get neither MAC address of both sides of communication nor length of the head of packet.

- For AMPDU packet, we can not get the count of packets or the length of packet.

- This structure contains: length of packet, rssi and FEC_CODING.

- RSSI and FEC_CODING are used to guess if the packets are sent from same device.

Summary

We should not take too long to process the packets. Otherwise, other packets may be lost. The diagram below shows the format of a ieee80211 packet:

- The first 24 bytes of MAC Header of data packet are needed:

  - Address 4 field depends on FromDS and ToDS which is in Frame Control;

  - QoS Control field depends on Subtype which is in Frame Control;

  - HT Control field depends on Order Field which is in Frame Control;

  - More details are found in IEEE Std 80211-2012.

- For WEP packets, MAC Header is followed by 4 bytes IV and before FCS there are 4 bytes ICV.

- For TKIP packet, MAC Header is followed by 4 bytes IV and 4 bytes EIV, and before FCS there are 8 bytes MIC and 4 bytes ICV.

- For CCMP packet, MAC Header is followed by 8 bytes CCMP header, and before FCS there are 8 bytes MIC.

## ESP8266 ADC & VDD33

SARADC in ESP8266 provides the following two applications, but cannot be used at the same time:

- Measure the power voltage of VDD3P3 pin 3 and 4：

The function to measure PA_VDD pin’s voltage: uint16 system_get_vdd33(void)；

- Measure the Input voltage of TOUT pin 6：

The function to measure the voltage of TOUT pin：uint16 system_adc_read(void). The described parameter of RF_init is esp_init_data_default.bin.

### Applications Scenario 1：Measure power voltage of VDD3P3 pin 3 and 4

Hardware design: TOUT pin must be suspended.

RF_init parameter：

The 107th byte in esp_init_data_default.bin（0〜～127byte）is “vdd33_const“ , and it must be set as 0xFF， that is 255；

RF Calibration work-in-progress：

Optimize the operating of RF circuit, after testing the power voltage of VDD3P3 pin 3 and 4.

User software：system_get_vdd33 is available；system_adc_read is disabled.

### Applications Scenario 2：Input Voltage of TOUT pin 6

Hardware design: wire TOUT pin to external circuitry，Input Voltage Range restricted to 0~1.0V.

RF_init parameter：

The 107th byte in esp_init_data_default.bin（0〜～127byte）is“vdd33_const”,and it must be set as the real power voltage of VDD3P3 pin 3 and 4.

The range of operating voltage of ESP8266 is 1.8V~3.6V，the unit of vdd33_const is 0.1V，so effective value range of vdd33_const is 18~36.

It will dynamically vary if the voltage is unstable, please input the Minimum*10 of the voltage’s variation of vdd33_const.

RF Calibration work-in-progress：

Optimize the operating of RF circuit, according to the value of 107th byte in“vdd33_const”, the tolerance is +-0.2V.

User software：system_get_vdd33 is disable；system_adc_read is available.

### Notice

Notice 1 The parameter of RF_ini, that is the 107th byte in esp_init_data_default.bin（0〜～127）

should be named as vdd33_const， and the explanations are as follows：

- Optimize the operating of RF circuit, after testing the power voltage of VDD3P3 pin 3 and 4 inside ESP8266 RF Calibration, when vdd33_const = 0xFF.

- Optimize the operating of RF circuit with （vdd33_const/10）inside ESP8266 RF Calibration,when 18 <= vdd33_const <= 36.

- Optimize the operating of RF circuit with the default value of 3.0V inside ESP8266 RF Calibration, when vdd33_const < 18 or 36 < vdd33_const < 255.

Notice 2 When measure the power voltage of VDD3P3 pin3 and 4 with function(system_get_vdd3) :

- Tout pin must be suspended, the 107th byte (vdd33_const) in RF_init parameter must be equal to 0xFF.

- Only the 107th byte (vdd33_const) in RF_init parameter is equal to 0xFF, can the return values from function(system_get_vdd33) are valid, or function(system_get_vdd33) returns to 0xFFFF.

- The value returned is 1/1024 V.

Notice 3 When measure the voltage of Tout pin 6 with function(system_adc_read):

- The 107th byte (vdd33_const) in RF_init parameter must be set as the real power voltage.

- Only the 107th byte (vdd33_const) in RF_init parameter isn’t equal to0xFF, can the return values from function(system_adc_read) are valid; or function(system_adc_read) returns to 0xFFFF.

- The value returned is 1/1024 V.

## ESP8266 soft-AP and station channel configuration

Even though ESP8266 can be in soft-AP + station mode，it actually has only one hardware channel.

So in soft-AP + station mode, ESP8266 soft-AP will adjust its channel configuration to be as same as ESP8266 station.

This limitation may cause some inconvenience in softAP + station mode users need to pay attention，for example:

Case 1.

- If user connect ESP8266 station to a router(e.g. router is in channel 6)

- Then set ESP8266 softAP by wifi_softap_set_config

- The API may return true, but channel will always be channel 6. Because we have only one hardware channel.

Case 2.

- If user set ESP8266 softAP a channel number(e.g. channel 5) by wifi_softap_set_config

- Some stations connected to ESP8266 softAP.

- Then connect ESP8266 station to a router of which channel number is different (e.g. channel 6) .

- ESP8266 softAP has to adjust its channel to be as same as ESP8266 station , in this case, is channel 6.

- So the stations that connected to ESP8266 softAP in step 2 will be disconnected because of the channel change.